Master Files

| Chapter Chair/Editor: | Mark Shafarman Oacis Healthcare Systems, Inc. |
| --- | --- |
| Editor: Staff and Practitioner, Location, and Charge Description | Francine Kitchen, PhD Software Technologies Corporation |
| Editor: Test/Observation | Clement J. McDonald, MD Regenstrief Institute and Indiana University School of Medicine |
| Editor: Clinical Trials | Susan Rea Welch M.D. Anderson Cancer Center |
| Editor: 2.3.1 | Cathy Wallace SMS |

## PURPOSE

In an open-architecture healthcare environment there often exists a set of common reference files used by one or more application systems.  Such files are called master files.  Some common examples of master files in the healthcare environment include:

a)	doctor master file

b)	system user (and password) master file

c)	location (census and clinic) master file

d)	device type and location (e.g., workstations, terminals, printers, etc.)

e)	lab test definition file

f)	exam code (radiology) definition file

g)	charge master file

h)	patient status master

i)	patient type master

These common reference files need to be synchronized across the various applications at a given site.  The Master Files Notification message provides a way of maintaining this synchronization by specifying a standard for the transmission of this data between applications.

In many implementations, one application system will “own” a particular master file such as the doctor master file.  The changes (e.g., adds, deletes, updates) to this file are made available to various other applications on a routine basis.  The Master Files Notification message supports this common case, but also supports the situation where an application not “owning” a particular master file, transmits update information to other systems (usually to the “owning” system), for review and possible inclusion.

The Master Files Notification message supports the distribution of changes to various master files between systems in either online or batch modes, and allows the use of either original or enhanced acknowledgment modes, as well as providing for a delayed application acknowledgment mode.  These messages use the MSH segment to pass the basic event code (master files notification or acknowledgment).  The MFI (master file identification) segment identifies the master file being updated as well as the initial and requested dates for “file-level” events (such as “replace file”).  For each record being changed, the MFE (Master File Entry) segment carries the record-level event code (such as add, update, etc.), the initial and requested dates for the event, and the record-level key identifying the entry in the master file.  The MFA (master file acknowledgment) segment returns record-specific acknowledgment information.

Note: The MFE segment is not the master file record, but only specifies its identifier, event, and event dates. The master file record so identified is contained in either Z-segments or HL7-defined segments immediately following the MFE segment. This record may be either a flat record contained in a single segment, or a complex record needing more than a single segment to carry its data and (usually hierarchical) structure.

The master file segments commonly needed across HL7 applications as well as those specific to the various application chapters, are defined in Sections 8.6, “STAFF AND PRACTITIONER MASTER FILES,”  through 8.10, “Clinical Trials MASTER FILES,” of this chapter.

A given master files message concerns only a single master file.  However, the location of a record-level event code (and requested activation date) on the MFE and the MFA segments allows a single message to contain several types of changes (events) to that file.

The Master Files Notification events do not specify whether the receiving system must support an automated change of the master file in question, nor do they specify whether the receiving system must create a file in the same form as that maintained on the sending system.

In general, the way in which the receiving system processes the change notification message will depend on both the design of the receiving system and the requirements negotiated at the site.  Some systems and/or sites may specify a manual review of all changes to a particular master file.  Some may specify a totally automated process.  Not every system at every site will need all the fields contained in the master file segment(s) following the MFE segment for a particular master file entry.

This also means that an application acknowledgment (or a deferred application acknowledgment) from a receiving system that it changed a particular record in its version of the master file does not imply that the receiving system now has an exact copy of the information and state that is on the sending system: it means only that whatever subset of that master file’s data (and state) that has been negotiated at the site is kept on the receiving system in such a manner that a new Master Files Notification transaction with the same primary key can be applied unambiguously (in the manner negotiated at the site) to that subset of information.

## TRIGGER EVENTS

The Master Files Change Notification message can be used for the following message-level trigger events:

Mnn:	A message containing notifications of changes to a single master file.

nn defines a particular HL7 master file.  Currently-defined values are (see HL7 table 0003 - Event type ): M01 - master file not otherwise specified (for backward compatibility only); M02 - staff/practitioner master file; M03 - test/observation master file; M04 - charge description master file; M05 - location master file; M06 - clinical study master file; M12 - M99 - reserved for future HL7-defined master files.  Site-specific master files should use a code of the form Znn. (See also Section 8.4.1.1, “Master file identifier  (CE)   00658,” HL7 table 0175 - Master file identifier codes.)

An MFN message may contain the following “file-level” events, as specified in the MFI segment:

REP:	Replace current version of this master file with the version contained in this message.

UPD:	Change file records as defined in the record-level event codes for each record that follows.

These are the only file-level events currently defined.  REP means that every MFE segment that follows will use the MAD event code.

The replace option allows the sending system to replace a file without sending delete record-level events for each record in that file.  UPD means that the events are defined according to the record-level event code contained in each MFE segment in that message.

An MFN message may contain the following “record-level” events, as specified in the MFE segments.

MAD:	Add record to master file.

MDL:	Delete record from master file.

MUP:	Update record for master file.

MDC:	Deactivate: discontinue using record in master file, but do not delete from database.

MAC:	Reactivate deactivated record.

The MFD transaction is used for the following trigger event:

MFA:	Master Files Delayed Application Acknowledgment.

## MESSAGES

The following messages are defined for master files transactions:  MFN, master files notification; MFK, master files application acknowledgment; MFD, master files delayed application acknowledgment; and MFQ, master files query.

### MFN/MFK - master files notification

The MFN transaction is defined as follows:

MFN					Master File Notification	Chapter

MSH					Message Header	2

MFI					Master File Identification	8

{MFE				Master File Entry	8

[Z..] }			One or more HL7 and/or	(varies)

Z-segments carrying the data

for the entry identified in the MFE segment

MFK					Master File Application Acknowledgment	Chapter

MSH					Message Header	2

MSA					Acknowledgment	2

[ ERR ]				Error	2

MFI					Master File Identification	8

{ [MFA] }			Master file ACK segment	8

The master file record identified by the MFE segment is contained in either Z-segments and/or HL7-defined segments immediately following the MFE segment, and is denoted by “Z...” in the MFN abstract message definition given above.  This record may be either a flat record contained in a single segment, or a complex record needing more than a single segment to carry its data and (usually hierarchical) structure.

The master file record “[Z..]” identified by the MFE segment is optional (indicated by square brackets) in the single case where the master file is a simple one which contains only a key and the text value of that key.  For this case only, both values may be carried in  MFE-4-primary key value.

Note: If the file-level event code is “REP” (replace file), then each MFA segment must have a record-level event code of “MAD” (add record to master file).

Note: The MFK message is used for an application acknowledgment in either the original or enhanced acknowledgment modes.

The MFA segment carries acknowledgment information for the corresponding MFE segment (identified by  MFA-5-primary key value).

### MFD/ACK - master files delayed application acknowledgment

The MFD transaction is the delayed application acknowledgment.  It can be used to return  “deferred” application-level acknowledgment statuses at the MFE level, without reference to the original MFN message.  It is defined as follows:

MFD					Master File Delayed Acknowledgment	Chapter

MSH					Message Header	2

MFI					Master File Identification	8

{ [MFA] }			Master file ACK segment	8

ACK					General Acknowledgment	Chapter

MSH					Message Header	2

MSA					Acknowledgment	2

[ ERR ]				Error	2

### MFQ/MFR - master files query

The MFQ transaction allows a system to query for a particular record or group records (defined by the primary key) in a particular master file.

The Master files query is defined as follows:

MFQ					Query for Master File Record	Chapter

MSH					Message Header	2

QRD					Query Definition	2

[QRF]				Query Filter	2

[DSC]				Continuation	2

MFR					Master Files Response	Chapter

MSH					Message Header	2

MSA					Acknowledgment	2

[ ERR ]				Error	2

QRD					Query Definition	2

[QRF]				Query Filter	2

MFI					Master File Name	8

{MFE				Master File Entry	8

[Z..] }			One or more HL7 and/or	(varies)

Z-segments carrying the data for the entry

identified in the MFE segment.

[DSC]				Continuation	2

#### MFQ use notes

The value “MFQ” of the QRD-what subject filter of the QRD segment identifies a master files query.  The QRD-what department data code of the QRD segment identifies the name of the master file in question.  The QRD-what data code value qual of the QRD segment identifies the primary key (or keys, or range of keys) defining the master file MFE segments (and associated master file records, denoted by “Z”) to be returned with the response.  The QRF segment may be used to define time ranges, particular MFN record-level event codes etc.  Unless otherwise specified, the response returns only active current record(s).

## general master file SEGMENTs

The following segments are defined for the master files messages.

### MFI - master file identification segment

The fields in the MFI segment are defined in Figure 8-1 - MFI attributes.

Figure 8-1.  MFI attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 60 | CE | R |  | 0175 | 00658 | Master File Identifier |
| 2 | 180 | HD | O |  |  | 00659 | Master File Application Identifier |
| 3 | 3 | ID | R |  | 0178 | 00660 | File-Level Event Code |
| 4 | 26 | TS | O |  |  | 00661 | Entered Date/Time |
| 5 | 26 | TS | O |  |  | 00662 | Effective Date/Time |
| 6 | 2 | ID | R |  | 0179 | 00663 | Response Level Code |

8.4.1.0	MFI field definitions

#### Master file identifier  (CE)   00658

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field is a CE data type that identifies a standard HL7 master file.  This table may be extended by local agreement during implementation to cover site-specific master files (z-master files).  Refer to HL7 table 0175 - Master file identifier code for valid values.

Table 0175 - Master file identifier code

| Value | Description |
| --- | --- |
| CDM | Charge description master file |
| CMA | Clinical study with phases and scheduled master file |
| CMB | Clinical study without phases but with scheduled master file |
| LOC | Location master file |
| OMA | Numerical observation master file |
| OMB | Categorical observation master file |
| OMC | Observation batteries master file |
| OMD | Calculated observations master file |
| PRA | Practitioner master file |
| STF | Staff master file |

#### Master files application identifier  (HD)   00659

Components:  <namespace ID (IS) ^ <universal ID (ST) ^ <universal ID type (ID)

Definition:  This field contains an optional code of up to 180 characters which (if applicable) uniquely identifies the application responsible for maintaining this file at a particular site.  A group of intercommunicating applications may use more than a single instance of a master file of certain type (e.g., charge master or physician master).  The particular instance of the file is identified by this field.

#### File-level event code  (ID)   00660

Definition:  This field defines the file-level event code.  Refer to HL7 table 0178 - File level event code for valid values.

Table 0178 - File level event code

| Value | Description |
| --- | --- |
| REP | Replace current version of this master file with the version contained in this message |
| UPD | Change file records as defined in the record-level event codes for each record that follows |

#### Entered date/time  (TS)   00661

Definition:  This field contains the time stamp for file-level event on originating system.

#### Effective date/time  (TS)   00662

Definition:  This optional field contains the effective date/time, which can be included for file-level action specified.  It is the date/time the originating system expects that the event is to have been completed on the receiving system.  If this field is not present, the action date/time should default to the current date/time (when the message is received).

#### Response level code  (ID)   00663

Definition:  These codes specify the application response level defined for a given Master File Message at the MFE segment level as defined in HL7 table 0179 - Response level.  Required for MFN-Master File Notification message.  Specifies additional detail (beyond MSH-15-accept acknowledgment type and MSH-16-application acknowledgment type) for application-level acknowledgment paradigms for Master Files transactions.  MSH-15-accept acknowledgment type and MSH-16-application acknowledgment type operate as defined in Chapter 2.

Table 0179 - Response level

| Value | Description |
| --- | --- |
| NE | Never. No application-level response needed |
| ER | Error/Reject conditions only. Only MFA segments denoting errors must be returned via the application-level acknowledgment for this message |
| AL | Always. All MFA segments (whether denoting errors or not) must be returned via the application-level acknowledgment message |
| SU | Success. Only MFA segments denoting success must be returned via the application-level acknowledgment for this message |

### MFE - master file entry segment

Figure 8-2.  MFE attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | ID | R |  | 0180 | 00664 | Record-Level Event Code |
| 2 | 20 | ST | C |  |  | 00665 | MFN Control ID |
| 3 | 26 | TS | O |  |  | 00662 | Effective Date/Time |
| 4 | 200 | Varies | R | Y |  | 00667 | Primary Key Value - MFE |
| 5 | 3 | ID | R |  | 0355 | 01319 | Primary Key Value Type |

8.4.2.0	MFE field definitions

#### Record-level event code  (ID)   00664

Definition:  This field defines the record-level event for the master file record identified by the MFI segment and the primary key field in this segment.   Refer to HL7 table 0180 - Record level event code for valid values.

Table 0180 - Record-level event code

| Value | Description |
| --- | --- |
| MAD | Add record to master file |
| MDL | Delete record from master file |
| MUP | Update record for master file |
| MDC | Deactivate: discontinue using record in master file, but do not delete from database |
| MAC | Reactivate deactivated record |

Note: If the file-level event code is “REP” (replace file), then each MFA segment must have a record-level event code of “MAD” (add record to master file).

#### MFN control ID  (ST)   00665

Definition:  A number or other identifier that uniquely identifies this change to this record from the point of view of the originating system.  When returned to the originating system via the MFA segment, this field allows the target system to precisely identify which change to this record is being acknowledged.  It is only required if the MFI response level code requires responses at the record level (any value other than NE).

Note: Note that this segment does not contain a Set ID field. The MFE-2-MFN control ID implements a more general concept than the Set ID. It takes the place of the SET ID in the MFE segment.

#### Effective date/time  (TS)   00662

Definition:  An optional effective date/time can be included for the record-level action specified.  It is the date/time the originating system expects that the event is to have been completed on the receiving system.  If this field is not present, the effective date/time should default to the current date/time (when the message is received).

#### Primary key value - MFE  (Varies)   00667

Definition:  This field uniquely identifies the record of the master file (identified in the  MFI segment) to be changed (as defined by the record-level event code).  The data type of field is defined by the value of MFE-5-value type, and may take on the format of any of the HL7 data types defined in HL7 table 0355 – Primary key value type.  The PL data type is used only on Location master transactions.

The following exception to the use of the CE data type is deprecated in v 2.3.1, and left only to satisfy backwards compatibility.  When the CE data type is used, the first component of this CE data field carries an optional subcomponent, the application ID, that uniquely identifies the application responsible for creating the primary key value.  The application ID subcomponent can be used to guarantee uniqueness of the primary key across multiple applications.

The repetition of the primary key permits the identification of an individual component of a complex record as the object of the record-level event code.  This feature allows the Master Files protocol to be used for modifications of single components of complex records.  If this field repeats, the field MFE-5-value type must also repeat (with the same number of repetitions), and the data type of each repetition of MFE-4-primary key value type is specified by the corresponding repetition of MFE-5-value type.

#### Primary key value type (ID)  01319

Definition: This field contains the HL7 data type of MFE-4-primary key value.  The valid values for the data type of a primary key are listed in HL7 table  0355 –  Primary key value type.

Table 0355 - Primary key value type

| Value | Description |
| --- | --- |
| PL | Person location |
| CE | Coded element |

### MFA - master file acknowledgment segment

The MFA segment contains the following fields as defined in Figure 8-3 - MFA attributes.

Figure 8-3.  MFA attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | ID | R |  | 0180 | 00664 | Record-Level Event Code |
| 2 | 20 | ST | C |  |  | 00665 | MFN Control ID |
| 3 | 26 | TS | O |  |  | 00668 | Event Completion Date/Time |
| 4 | 60 | CE | R |  | 0181 | 00669 | MFN Record Level Error Return |
| 5 | 60 | CE | R | Y |  | 01308 | Primary Key Value – MFA |
| 6 | 3 | ID | R |  | 0355 | 01320 | Primary Key Value Type - MFA |

8.4.3.0	MFA field definitions

#### Record-level event code  (ID)   00664

Definition:  This field defines record-level event for the master file record identified by the MFI segment and the primary key in this segment.  Refer to HL7 table 0180 - Record level event code for valid values.

#### MFN control ID  (ST)   00665

Definition:  This field contains a number or other identifier that uniquely identifies this change to this record from the point of view of the originating system.  This field uniquely identifies the particular record (identified by the MFE segment) being acknowledged by this MFA segment.  When returned to the originating system via the MFA segment, this field allows the target system to precisely identify which change to this record is being acknowledged.  It is only required if MFI-6-response level code requires responses at the record level (any value other than NE).

#### Event Completion date/time  (TS)   00668

Definition:  This field may be required or optional depending on the site specifications for the given master file, master file event, and receiving facility.

#### MFN Record Level error return   (CE)   00669

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the status of the requested update.  Site-defined table, specific to each master file being updated via this transaction.

Refer to user-defined table 0181 - MFN record level error return for suggested values.  All such tables will have at least the following two return code values:

User-defined Table 0181 - MFN record-level error return

| Value | Description |
| --- | --- |
| S | Successful posting of the record defined by the MFE segment |
| U | Unsuccessful posting of the record defined by the MFE segment |

#### Primary key value - MFA (CE)   01308

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field uniquely identifies the record of the master file (identified in the MFI segment) to be change status is being acknowledged (as defined by the field MFN-4-record level error return).  The data type of this field is defined by the value of MFA-6-value type-MFA, and may take on the format of any of the HL7 data types defined in HL7 table 0355 - Primary key value type.  The PL data type is used only on location master transactions.

The following exception to the use of the CE data type is deprecated in V2.3.1, and left in for backward compatibility.  When the CE data type is used, the first component of this CE data field carries an optional subcomponent, the application ID, that uniquely defines the application responsible for creating the primary key value.  The application ID subcomponents can be used to guarantee uniqueness of the primary key across multiple applications.

The repetition of the primary key permits the identification of an individual component of a complex record as the object of the record-level event code.  This feature allows the Master Files protocol to be used for modifications of single components of complex records.  If this field repeats, the field MFA-6-primary key value type-MFA must also repeat (with the same number of repetitions), and the data type of each repetition of MFA-5-primary key value-MFA is specified by the corresponding repetition of MFA-6-value type-MFA.

#### Primary key value type - MFA (ID)  01320

Definition: This field contains the HL7 data type of MFA-5-primary key value- MFA.  The valid  HL7 data types are listed in HL7 table  0355 –  Primary key value type.

Table 0355 – Primary key value type

| Value | Description |
| --- | --- |
| PL | Person location |
| CE | Coded element |

## GENERIC MASTER FILE EXAMPLES

This is an example of a proposed generic method of updating a standard HL7 table.  This particular example shows two records being added to HL7 table 0006-Religion.

Note: A standard HL7 table segment can be constructed by defining two fields: a table entry field (as a CE field) and a display-sort-key field (a numeric field) as follows.

### ZL7 segment (proposed example only)

Figure 8-4.  ZL7 attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 2 | 60 3 | CE NM | R R |  | xxxx xxxx | xxxxx xxxxx | HL7 table entry for table xxxx Display-sort-key |

8.5.1.0	ZL7 field definitions

#### HL7 table entry for table xxxx  (CE)   xxxxx

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains HL7 table values for identifier and text encoded as a CE data type.

#### Display-sort-key  (NM)   xxxxx

Definition:  This field is used to specify a non-alphabetic ordering for display or print versions of a standard HL7 table.

### MFN message with original acknowledgment mode

MSH|^~\&|HL7REG|UH|HL7LAB|CH|19910918060544||MFN^M01|MSGID002|P|2.3.1

MFI|0006^RELIGION^HL7||UPD|||AL

MFE|MAD|199109051000|199110010000|U^Buddhist^HL7|CE

ZL7|U^Buddhist^HL7|3^^Sortkey

MFE|MAD|199109051015|199110010000|Z^Zen Buddhist^HL7|CE

ZL7|Z^Zen Buddhist^HL7|12^^Sortkey

In this case, the primary key contains all the data needed for this simple table, so that the HL7 segment could be constructed with ONLY the single field, “sort-key,” rather than repeating the primary key value as we have done in this example.

MFK, master file application acknowledgment, as original mode acknowledgment of the HL7 message according to MFI Response Level Code of “AL.”

MSH|^~\&|HL7LAB|CH|HL7ADT|UH|19910918060546||MFK|MSGID99002|P|2.3.1

MSA|AA|MSGID002

MFI|0006^RELIGION^HL7||UPD||AL

MFA|MAD|199109051000|19910918060545|S|U^Buddhist^HL7|CE

MFA|MAD|199109051015|19910918060545|S|Z^Zen Buddhist^HL7|CE

### Enhanced mode application-level acknowledgment to the MFN message

#### Initial message with accept acknowledgment

MSH|^~\&|HL7REG|UH|HL7LAB|CH|19910918060544||MFN^M01|MSGID002|P|2.3.1||AL|AL

MFI|0006^RELIGION^HL7||UPD|||AL

MFE|MAD|199109051000|199110010000|U^Buddhist^HL7|CE

ZL7|U^Buddhist^HL7|3^^Sortkey

MFE|MAD|199109051015|199110010000|Z^Zen Buddhist^HL7|CE

ZL7|Z^Zen Buddhist^HL7|12^^Sortkey

MSH|^~\&|HL7LAB|CH|HL7ADT|UH|19910918060545||MSA|MSGID99002|P|2.3.1

MSA|CA|MSGID002

#### Enhanced mode application acknowledgment message

MSH|^~\&|HL7LAB|CH|HL7ADT|UH|19911001080504||MFK|MSGID99502|P|2.3.1||AL|

MSA|AA|MSGID002

MFI|0006^RELIGION^HL7||UPD||AL

MFA|MAD|199109051000|19910918010040|S|U^Buddhist^HL7|CE

MFA|MAD|199109051015|19910918010040|S|Z^Zen Buddhist^HL7|CE

MSH|^~\&|HL7ADT|UH|HL7LAB|CH|19911001080507||ACK|MSGID444|P|2.3.1

MSA|CA|MSGID5002

### Delayed application-level acknowledgment

#### Initial message with accept acknowledgment

MSH|^~\&|HL7REG|UH|HL7LAB|CH|19910918060544||MFN^M01|MSGID002|P|2.2||AL|NE

MFI|0006^RELIGION^HL7||UPD|||AL

MFE|MAD|199109051000|199110010000|U^Buddhist^HL7

ZL7|U^Buddhist^HL7|3^^Sortkey

MFE|MAD|199109051015|199110010000|Z^Zen Buddhist^HL7

ZL7|Z^Zen Buddhist^HL7|12^^Sortkey

MSH|^~\&|HL7LAB|CH|HL7ADT|UH|19910918060545||ACK|MSGID99002|P|2.2

MSA|CA|MSGID002

#### Deferred application acknowledgment message

MSH|^~\&|HL7LAB|CH|HL7ADT|UH|19910919060545||MFD|MSGID99002|P|2.2|||AL

MFI|0006^RELIGION^HL7||UPD|||AL

MFA|MAD|199109051000|19910919020040|S|U^Buddhist^HL7

MFA|MAD|199109051015|19910919020040|S|Z^Zen Buddhist^HL7

MSH|^~\&|HL7ADT|UH|HL7LAB|CH|19910919060546||ACK|MSGID444|P|2.2

MSA|CA|MSGID500

## STAFF AND PRACTITIONER MASTER FILES

### MFN/MFK - staff/practitioner master file message

The staff (STF) and practitioner (PRA) segments can be used to transmit master files information between systems. The STF segment provides general information about personnel; the PRA segment provides detailed information for a staff member who is also a health practitioner. Other segments may be defined to follow the STF segment to provide additional detail information for a particular type of staff member: the PRA segment is the first such segment.  When the STF and PRA segments are used in an MFN message, the abstract definition is as follows:

MFN			Master File Notification for Staff/Practitioner	Chapter

MSH					Message Header	2

MFI					Master File Identification	8

{MFE				Master File Entry	8

STF				Staff Identification	8

[PRA]			Practitioner Detail	8

}

MFK					Master File Acknowledgment	Chapter

MSH					Message Header	2

MSA					Acknowledgment	2

MFI					Master File Identification	8

[{MFA}]			Master File ACK segment	8

When the STF and PRA segments are used in the MFR message, the part of the message represented by:

{MFE

[Z..]}

is replaced by:

{MFE

STF

[PRA]

}

### STF - staff identification segment

The STF segment can identify any personnel referenced by information systems.  These can be providers, staff, system users, and referring agents.  In a network environment, this segment can be used to define personnel to other applications; for example, order entry clerks, insurance verification clerks, admission clerks, as well as provider demographics. MFE-4-primary key value is used to link all the segments pertaining to the same master file entry.  Therefore, in the MFE segment, MFE-4-primary key value must be filled in.  Other segments may follow the STF segment to provide data for a particular type of staff member. The PRA segment (practitioner) is one such.  It may optionally follow the STF segment in order to add practitioner-specific data. Other segments may be defined as needed.

Figure 8-5.  STF attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 60 | CE | R |  |  | 00671 | Primary Key Value - STF |
| 2 | 60 | CX | O | Y |  | 00672 | Staff ID Code |
| 3 | 48 | XPN | O | Y |  | 00673 | Staff Name |
| 4 | 2 | IS | O | Y | 0182 | 00674 | Staff Type |
| 5 | 1 | IS | O |  | 0001 | 00111 | Sex |
| 6 | 26 | TS | O |  |  | 00110 | Date/Time Of Birth |
| 7 | 1 | ID | O |  | 0183 | 00675 | Active/Inactive |
| 8 | 200 | CE | O | Y | 0184 | 00676 | Department |
| 9 | 200 | CE | O | Y | 0069 | 00677 | Hospital Service |
| 10 | 40 | XTN | O | Y |  | 00678 | Phone |
| 11 | 106 | XAD | O | Y |  | 00679 | Office/Home Address |
| 12 | 26 | CM | O | Y |  | 00680 | Institution Employment Start |
| 13 | 26 | CM | O | Y |  | 00681 | Institution Employment End |
| 14 | 60 | CE | O | Y |  | 00682 | Backup Person ID |
| 15 | 40 | ST | O | Y |  | 00683 | E-Mail Address |
| 16 | 200 | CE | O |  | 0185 | 00684 | Preferred Method Of Contact |
| 17 | 1 | IS | O |  | 0002 | 00119 | Marital Status |
| 18 | 20 | ST | O |  |  | 00785 | Job Title |
| 19 | 20 | JCC | O |  | 0327/0328 | 00786 | Job Code/Employee Classification |
| 20 | 2 | IS | O |  | 0066 | 01276 | Employment Status |
| 21 | 1 | ID | O |  | 0136 | 01275 | Additional Insured on Auto |
| 22 | 25 | DLN | O |  |  | 01302 | Driver’s License Number - Staff |
| 23 | 1 | ID | O |  | 0136 | 01229 | Copy Auto Ins |
| 24 | 8 | DT | O |  |  | 01232 | Auto Ins. Expires |
| 25 | 8 | DT | O |  |  | 01298 | Date Last DMV Review |
| 26 | 8 | DT | O |  |  | 01234 | Date Next DMV Review |

8.6.2.0   STF field definitions

#### Primary key value - STF  (CE)   00671

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field must match MFE-4-primary key value to identify which entry is being referenced.

#### Staff ID code  (CX)   00672

Components:  <ID (ST)> ^ <check digit (ST)> ^ <code identifying the check digit scheme employed (ID)> ^ <assigning authority (HD)> ^ <identifier type code (IS)> ^ <assigning facility (HD)>

Subcomponents of assigning authority:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Subcomponents of assigning facility:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Definition:  This field contains a personnel identification code or institution user number, used by the institution to identify this person.  Repeating field allows multiple ID codes per person, with the type of ID code indicated in the third component of the coded entry data type.

#### Staff name  (XPN)   00673

Components:  <family name (ST)> & <last_name_prefix (ST)> ^ <given name (ST)> ^ <middle initial or name (ST)> ^ <suffix (e.g., JR or III) (ST)> ^ <prefix (e.g., DR) (ST)> ^ <degree (e.g., MD) (IS)> ^ <name type code (ID) > ^ <name representation code (ID)>

Definition:  This field contains the staff person’s name.

#### Staff type  (IS)   00674

Definition:  This field contains a code identifying what type of staff.  Refer to user-defined table 0182 - Staff type for suggested values.  Values may include codes for staff, practitioner (physician, nurse, therapist, etc.), referral agent or agency, etc.

#### Sex  (IS)   00111

Definition:  This field contains the staff person’s sex.  Refer to user-defined table 0001 - Sex for suggested values.

#### Date/time of birth  (TS)   00110

Definition:  This field contains a staff member’s date and time of birth.

#### Active/inactive  (ID)   00675

Definition:  This field indicates whether person is currently a valid staff member.  Refer to HL7 table 0183 - Active/inactive for valid values.

Table 0183 - Active/inactive

| Value | Description |
| --- | --- |
| A | Active Staff |
| I | Inactive Staff |

#### Department  (CE)   00676

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the institution department to which this person reports or belongs.  Refer to user-defined table 0184 - Department for suggested values.

#### Service  (CE)   00677

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the hospital or ancillary service with which this staff person is associated.  Refer to user-defined table 0069 - Hospital service for suggested values.

#### Phone  (XTN)   00678

Components:  [NNN] [(999)]999-9999 [X99999] [B99999] [C any text] ^ <telecommunication use code (ID)> ^ <telecommunication equipment type (ID)> ^ <email address (ST)> ^ <county code (NM)> ^ <area/city code (NM)> ^ <phone number (NM) ^ <extension (NM)> ^ <any text (ST)>

Definition:  This field contains the staff person’s phone number.  This is a repeating field with a component for indicating which phone number is which. It is recommended that the last part of the XTN, [C any text], start with a code from the table associated below with STF-16-preferred method of contact, in order to indicate the type of each phone number in this repeating field.

#### Office/home address  (XAD)   00679

Components:  <street address (ST)> ^ <other designation (ST)> ^ <city (ST)> ^ <state or province (ST)> ^ <zip or postal code (ST)> ^ <country (ID)> ^ <address type (ID)> ^ <other geographic designation (ST)> ^ <county/parish code (IS)> ^ <census tract (IS)> ^ <address representation code (ID)> ^ <address representation code (ID)>

Definition:  This field contains the office address and home address of the staff person.  This is a repeating field.

#### Institution employment start (CM)   00680

Components:	<date (TS)> ^ <institution name (CE)>

Subcomponents for institution name:  <identifier (ST)> & <text (ST) & <name of coding system (ST)> & <alternate identifier (ST)> & <alternate text (ST)> & <name of alternate coding system (ST)>

Definition:  This field contains the date when staff became active for an institution.  Repeats.

#### Institution employment end (CM)   00681

Components: <date (TS)> ^ <institution name (CE)>

Subcomponents for institution name:  <identifier (ST)> & <text (ST) & <name of coding system (ST)> & <alternate identifier (ST)> & <alternate text (ST)> & <name of alternate coding system (ST)>

Definition:  This field contains the date when staff became active for an institution.  Repeats.

#### Backup person ID  (CE)   00682

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the  MFE-4-primary key value of the master file entry which corresponds to the designated backup person for this staff person.

#### E-mail address  (ST)   00683

Definition:  This field has been retained for backward compatibility.  (It is now present in the fourth component of STF-10-phone).

#### Preferred method of contact  (CE)   00684

Definition:  This field contains a one-letter code that indicates which of multiple phone numbers is the preferred method of contact for this person.  Note that all values of this code refer to this segment’s phone field, except for the value “E,” which refers to the E-mail address field.  If more than one phone number of the preferred type exists in STF-10-phone, this field refers to the first such instance.  Refer to HL7 table 0185 - Preferred method of contact for valid values.

Table 0185 - Preferred method of contact

| Value | Description |
| --- | --- |
| H | Home Phone Number |
| O | Office Phone Number |
| F | FAX Number |
| C | Cellular Phone Number |
| B | Beeper Number |
| E | E-Mail Address (for backward compatibility) |

#### Marital status (IS)   00119

Definition:  This field contains the staff member’s marital status.  Refer to user-defined table 0002 - Marital status for suggested values.  Same values as those for PID-16-marital status.

#### Job title (ST)   00785

Definition:  This field contains a descriptive name of the staff member’s  occupation (e.g., Sr. Systems Analyst, Sr. Accountant).

#### Job code/class  (JCC)   00786

Components:	<job code (IS)> ^ <job class (IS)>

Definition: This field contains the staff member’s  job code and employee classification.  Refer to user-defined table 0327 - Job code and user-defined table 0328 - Employee classification.

#### Employment status (IS)   01276

Definition:  This field contains the code that indicates the staff member’s employment status, e.g., full-time, part-time, self-employed, etc.  Refer to user-defined table 0066 - Employment status for suggested values.

#### Additional insured on auto (ID)   01275

Definition:  This field contains an indicator for whether the present institution is named as an additional insured on the staff member’s auto insurance, especially for use when staff is a driver for the institution.   Refer to HL7 table 0136 - Yes/no indicator for valid values.

Y	indicates that the institution is named as an additional insured

N	indicates that the institution is not named as an additional insured

#### Driver’s license number - staff (DLN)   01302

Components:  <license number (ST)> ^ <issuing state, province, country (IS)> ^ <expiration date (DT)

Definition:  This field contains the driver’s license information of staff, especially for use when staff is a driver for the institution.  For state or province refer to official postal codes for that country; for country refer to ISO 3166 for codes.

#### Copy  auto ins (ID)   01229

Definition:  This field contains an indicator for whether the institution has on file a copy of the staff member’s auto insurance, especially for use when staff is a driver for the institution.   Refer to HL7 table 0136 - Yes/no indicator for valid values.

Y	indicates that the institution has a copy on file

N	indicates that the institution does not have a copy on file

#### Auto ins. expires (DT)   01232

Definition:  This field contains the date on which the staff member’s driver’s license expires, especially for use when staff is a driver for the institution.

#### Date last DMV review (DT)   01298

Definition:  This field contains the date of the staff member’s most recent Department of Motor Vehicles review, especially for use when staff is a driver for the institution.

#### Date next DMV review (DT)   01234

Definition:  This field contains the date of the staff member’s next Department of Motor Vehicles review, especially for use when staff is a driver for the institution.

### PRA - practitioner detail segment

The PRA segment adds detailed medical practitioner information to the personnel identified by the STF segment.  A PRA segment may optionally follow an STF segment.  A PRA segment must always have been preceded by a corresponding STF segment.  The PRA segment may also be used for staff who work in healthcare who are not practitioners, but need to be certified, e.g., “medical records staff.”

Figure 8-6.  PRA attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 60 | CE | R |  |  | 00685 | Primary Key Value - PRA |
| 2 | 60 | CE | O | Y | 0358 | 00686 | Practitioner Group |
| 3 | 3 | IS | O | Y | 0186 | 00687 | Practitioner Category |
| 4 | 1 | ID | O |  | 0187 | 00688 | Provider Billing |
| 5 | 100 | CM | O | Y | 0337 | 00689 | Specialty |
| 6 | 100 | CM | O | Y | 0338 | 00690 | Practitioner ID Numbers |
| 7 | 200 | CM | O | Y |  | 00691 | Privileges |
| 8 | 8 | DT | O |  |  | 01296 | Date Entered Practice |

8.6.3.0   PRA field definitions

#### Primary key value - PRA  (CE)   00685

Definition:  This field must match MFE-4-primary key value, to identify which entry is being referenced.

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

#### Practitioner group  (CE)   00686

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the name and/or code of a group of practitioners to which this practitioner belongs.  Refer to user-defined table 0358  Practitioner group for suggested values.

#### Practitioner category  (IS)   00687

Definition:  This field contains the category of practitioner.  Refer to user-defined table 0186 - Practitioner category whose values may include codes for staff physician, courtesy physician, resident, physician assistant, physical therapist, psychiatrist, psychologist, pharmacist, registered nurse, licensed practical nurse, licensed vocational nurse, nurse practitioner, etc.

#### Provider billing  (ID)   00688

Definition:  This field indicates how provider services are billed.  Refer to HL7 table 0187 - Provider billing for valid values.

Table 0187 - Provider billing

| Value | Description |
| --- | --- |
| P | Provider does own billing |
| I | Institution bills for provider |

#### Specialty  (CM)   00689

Components:  <specialty name (ST)> ^ <governing board (ST)> ^ <eligible or certified (IS)> ^ <date of certification (DT)>

Definition:  This repeating field is made up of multiple components to record the practitioner’s specialties.  The multiple components of each specialty are: (1) specialty name or abbreviation, identifies provider’s specialty, (2) name of specialty governing board, (3) Certification Status, (4) certified date contains the  date of certification, if certified.

Table - 0337 - Certification Status

| Value | Description |
| --- | --- |
| E | Eligible |
| C | Certified |

#### Practitioner ID numbers  (CM)   00690

Components:  <ID number (ST)> ^ <type of ID number (IS)> ^ <state/other qualifying info (ST)> ^ <expiration date>

Definition:  This repeating field contains this practitioner’s license numbers and other ID numbers.  This is a field made up of the following components: (1) the ID number, and (2) the type of number, and optionally (3) the state or province in which it is valid, if relevant, or other qualifying information.  It is recommended that state qualifications use the abbreviations from the postal service of the country.  The practitioner ID number type (component 2) is a user-defined table (table 0338).

User-defined Table 0338 - Practitioner ID number type

| Value | Description |
| --- | --- |
| UPIN | Unique Physician ID. No. |
| SL | State License Number |
| MCD | Medicaid Number |
| GL | General Ledger Number |
| CY | County Number |
| TAX | Tax ID Number |
| DEA | Drug Enforcement Agency No. |
| MCR | Medicare Number |
| L&I | Labor and Industries Number |
| QA | QA Number |
| TRL | Training License Number |

#### Privileges  (CM)   00691

Components:  <privilege (CE)> ^ <privilege class (CE)> ^ <expiration date (DT)> ^ <activation date (DT)> ^ <facility (EI)>

Subcomponents for privilege:  < identifier (ID)> & <text (ST)> & <name of coding system (ST)> & <alternate identifier (ID)> & <text (ST)> & <name of alternate coding system(ST)>

Subcomponents for privilege class:  < identifier (ID)> & <text (ST)> & <name of coding system (ST)> & <alternate identifier (ID)> & <text (ST)> & <name of alternate coding system(ST)>

Subcomponents for facility:  < entity identifier (ST)> ^ <namespace ID (IS)> ^ <universal ID> ^ <universal ID type (ID)>

Definition:  This field contains the institutional privileges which this provider may exercise.  Depends upon institutional needs.  For example, admit, transfer, discharge, place orders, verify orders, review results, etc.  Can also be used for privileges other than patient services.  This is a repeating field, with each privilege made up of the following components: (1) privilege; (2) privilege class; (3) privilege expiration date, if any; (4) privilege activation date, if any, and (5) facility.  Note that the privilege and privilege class components are CE data types, and thus they are encoded with the subcomponent delimiter (&) rather than the component delimiter (^).  The facility component is an EI data type specifying the facility to which the privilege applies and is encoded with the subcomponent delimiter (&) rather than the component delimiter (^).

#### Date entered practice  (DT)   01296

Definition:  This field contains the date the practitioner began practicing at the present institution (e.g., at hospital, at physician organization, at managed care network).

### Example:  doctor master file MFN message

MSH|^~\&|HL7REG|UH|HL7LAB|CH|19910918060544||MFN^M02|MSGID002|P|2.3|||AL|NE

MFI|0004^DOCTOR^HL7||UPD|||AL

MFE|MAD|U2246|199110011230|PMF98123789182^^PLW

STF|PMF98123789182^^PLW|U2246^^^PLW~111223333^^^USSSA^SS|KILDARE^RICHARD^J^JR^DR^M.D.|P|M|19511004|A|^ICU|^MED|(206)689-1999X345CO~(206)283-3334CH(206)689-1345X789CB|214 JOHNSON ST^SUITE 200^SEATTLE^WA^98199^H~3029 24TH AVE W^^SEATTLE, WA^98198^O |19890125^UMC&University Medical Center&L01||PMF88123453334|74160.2326@COMPUSERV.COM|B

PRA|PMF98123789182^^PLW|^KILDARE FAMILY PRACTICE|ST|I|OB/GYN^STATE BOARD OF OBSTETRICS AND GYNECOLOGY^C^19790123|1234887609^UPIN~1234987^CTY^MECOSTA~223987654^TAX~1234987757^DEA~12394433879^MDD^CA|ADMIT&&ADT^MED&&L2^19941231~DISCH&&ADT^MED&&L2^19941231|

## TEST/OBSERVATIONS MASTER FILES

### General approach of test/observation master files

These segments define the format for the general information about the observations that a clinical or diagnostic service produces and sends to its “clients.”  This format can be used to send the producer’s entire test/observation definition or a few of the producer’s observations, such as those with procedure, technique, or interpretation changes.

In anticipation of an object-oriented organization of segments in future releases of this Standard, the attributes of observations/batteries have been grouped into six different segments:

OM1 	contains the  attributes that apply to all observations

OM2 	applies to numerically-valued observations

OM3 	applies to text or code-valued observations

OM4 	applies to observations or batteries that require specimens

OM5 	contains the  attributes of batteries, or sets of observations or other batteries

OM6 	contains the  quantities (observations in a most general sense) that are calculated from one or more other observations

Thus, the full definition of a numerically-valued laboratory observation would require the transmission of OM1, OM2, and OM4.

In the following discussion, we use OMx to refer to any of the six observation-defining segments.  Each instance of an OMx segment contains the information about one observation or observation battery.  These OMx segments are designed to be “inclusive” and accommodate the attributes of many kinds of observations.  Thus, the fact that a field is listed in a particular segment should not be construed as meaning that a producer must include information about that item in its definition transmission.  Many fields will apply to some terms; others will not.  One observation producer may choose to populate one set of fields; another may choose to populate a different set of fields, according to the requirements of that producer’s “client.”

Most of the fields of data type TX in those segments are intended to include information typically contained in a diagnostic service’s user manual.  Such fields should describe how the data is to be interpreted or used, and are not intended for computer interpretation.

Remember that the magnitude of a treatment can also be regarded as an observation and, as such, can be represented as an observation within these segments.  Many examples exist.  When a blood gas is transmitted, the requesting service usually transmits the amount of inspired O2 (a treatment) on requisition.  (In an electronic transmission, the service would send this as an OBX segment, along with the electronic order for the test.)  When blood levels are drawn, the amount and time of the last dose are routinely included as observations on the request for service.  A pharmacy system could routinely send to a medical record system the average daily dose of each outpatient medication it dispenses.  In such cases, the treatment amounts would be observations to the receiving system and would be transmitted as OBX segments.  When received, they would be treated like any other observation.  A medical record system could then create, for example, a flowchart of lab results, or lab results mixed with relevant treatments.

### MFN/MFR - test/observation master file

The usage of the OMx segments in the Master Files MFN and MFR messages is described in Sections 8.3.1, “MFN/MFK - master files notification,”  and 8.3.3, “MFQ/MFR - master files query,” above.  Basically the segment groupings described below follow the MFI and MFE segments in those messages (replacing the [Z...] section as follows:

MFN					Master File Notification	Chapter

MSH					Message Header	2

MFI					Master File Identification	8

{MFE				Master File Entry	8

OM1				General Segment (Fields That Apply	8

to Most Observations)

[other segments(s)]

}

where other segments can be any of the following combinations:

MFI-1-master file identifier = OMA, for numeric observations (second component of MSH-9-message  type = M08).

[

[OM2]	    Numeric Observation Segment

[OM3]	    Categorical Test/Observation Segment

[OM4]	    Observations that Require Specimens

]

or

MFI-1-master file identifier = OMB, for categorical observations (second component of MSH-9- message  type = M09).

[OM3		Categorical Test/Observation Segment

[{OM4}]	Observations that Require Specimens

]

or

MFI-1-master file identifier = OMC, for observation batteries (second component of MSH-9-message type = M10).

[OM5		Observation Batteries

[{OM4}]	Observations that Require Specimens

]

or

MFI-1-master file identifier = OMD, calculated observations (second component of MSH-9-message type = M11).

[OM6		Observations Calculated from Other Observations

OM2]		Numeric Observation Segment

Note: A test/observation definition may have both an OM2 (numeric) and OM3 (categorical) segment included in case the value may be either numeric and/or categorical.

### OM1 - general segment (fields that apply to most observations)

The OM1 segment contains the attributes that apply to the definition of most observations.  This segment also contains the field attributes that specify what additional segments might also be defined for this observation.

Figure 8-7.  OM1 attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 | NM | R |  |  | 00586 | Sequence Number – Test/Observation Master File |
|  | 200 | CE | R |  |  | 00587 | Producer's Test/Observation ID |
|  | 12 | ID | O | Y | 0125 | 00588 | Permitted Data Types |
|  | 1 | ID | R |  | 0136 | 00589 | Specimen Required |
|  | 200 | CE | R |  |  | 00590 | Producer ID |
|  | 200 | TX | O |  |  | 00591 | Observation Description |
|  | 200 | CE | O |  |  | 00592 | Other Test/Observation IDs for the Observation |
|  | 200 | ST | R | Y |  | 00593 | Other Names |
|  | 30 | ST | O |  |  | 00594 | Preferred Report Name for the Observation |
| 10 | 8 | ST | O |  |  | 00595 | Preferred Short Name or Mnemonic for Observation |
| - 1 | 200 | ST | O |  |  | 00596 | Preferred Long Name for the Observation |
| 12 | 1 | ID | O |  | 0136 | 00597 | Orderability |
| 13 | 60 | CE | O | Y |  | 00598 | Identity of Instrument Used to Perform this Study |
| 14 | 200 | CE | O | Y |  | 00599 | Coded Representation of Method |
| 15 | 1 | ID | O |  | 0136 | 00600 | Portable |
| 16 | 1 | CE | O | Y |  | 00601 | Observation Producing Department/Section |
| 17 | 40 | XTN | O |  |  | 00602 | Telephone Number of Section |
| 18 | 1 | IS | R |  | 0174 | 00603 | Nature of Test/Observation |
| 19 | 200 | CE | O |  |  | 00604 | Report Subheader |
| 20 | 20 | ST | O |  |  | 00605 | Report Display Order |
| 21 | 26 | TS | O |  |  | 00606 | Date/Time Stamp for any change in Definition for the Observation |
| 22 | 26 | TS | O |  |  | 00607 | Effective Date/Time of Change |
| 23 | 20 | NM | O |  |  | 00608 | Typical Turn-Around Time |
| 24 | 20 | NM | O |  |  | 00609 | Processing Time |
| 25 | 40 | ID | O | Y | 0168 | 00610 | Processing Priority |
| 26 | 5 | ID | O |  | 0169 | 00611 | Reporting Priority |
| 27 | 200 | CE | O | Y |  | 00612 | Outside Site(s) Where Observation may be Performed |
| 28 | 1000 | XAD | O | Y |  | 00613 | Address of Outside Site(s) |
| 29 | 400 | XTN | O |  |  | 00614 | Phone Number of Outside Site |
| 30 | 1 | IS | O |  | 0177 | 00615 | Confidentiality Code |
| 31 | 200 | CE | O |  |  | 00616 | Observations Required to Interpret the Obs |
| 32 | 64K | TX | O |  |  | 00617 | Interpretation of Observations |
| 33 | 64K | CE | O |  |  | 00618 | Contraindications to Observations |
| 34 | 200 | CE | O | Y |  | 00619 | Reflex Tests/Observations |
| 35 | 80 | TX | O |  |  | 00620 | Rules that Trigger Reflex Testing |
| 36 | 64K | CE | O |  |  | 00621 | Fixed Canned Message |
| 37 | 200 | TX | O |  |  | 00622 | Patient Preparation |
| 38 | 200 | CE | O |  |  | 00623 | Procedure Medication |
| 39 | 200 | TX | O |  |  | 00624 | Factors that may Effect the Observation |
| 40 | 60 | ST | O | Y |  | 00625 | Test/Observation Performance Schedule |
| 41 | 64K | TX | O |  |  | 00626 | Description of Test Methods |
| 42 | 60 | CE | O |  | 0254 | 00937 | Kind of Quantity Observed |
| 43 | 60 | CE | O |  | 0255 | 00938 | Point Versus Interval |
| 44 | 200 | TX | O |  | 0256/0257 | 00939 | Challenge Information |
| 45 | 200 | CE | O |  | 0258 | 00940 | Relationship Modifier |
| 46 | 200 | CE | O |  |  | 00941 | Target Anatomic Site Of Test |
| 47 | 200 | CE | O |  | 0259 | 00942 | Modality Of Imaging Measurement |

- 8.7.3.0 OM1 field definitions

#### Sequence number – test/observation master file (NM)   00586

Definition:  This field contains the first OM1 segment in a message and is described as 1, the second as 2, and so on.

#### Producer's test/observation ID  (CE)   00587

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the producer’s usual or preferred identification of the test or observation.   Only three components should be included: <ID code>^<service text name/description>^<source list of code>.  All components should be non-null.  The source list may be any of those included in ASTM Tables 3 and 5, or a local code.

#### Permitted data types  (ID)   00588

Definition:  This field contains the allowed data type(s) for this observation.  The codes are the same as those listed for OBX (a given observation may, under different circumstances, take on different data types). Indeed, under limited circumstances, an observation can consist of one or more fragments of different data types. When an observation may have more than one data type, e.g., coded (CE) and numeric (NM) the allowable data types should be separated by repeat delimiters.  Refer to HL7 table 0125 - Value type for valid values.

#### Specimen required  (ID)   00589

Definition:  This field contains a flag indicating whether or not at least one specimen is required for the test/observation.  Refer to HL7 table 0136 - Yes/no indicator as defined in Chapter 2.

Y 	one or more specimens are required to obtain this observation

N 	a specimen is not required

When a specimen is required, segment OM4 will usually be included (one per specimen is required).

#### Producer ID  (CE)   00590

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field uniquely identifies the service producing the observation described in this segment.  Three components should be included:  an identifying code, the name of the producer, and the identity of the coding system (e.g., 323-5678^Acme Special Lab^MC).  The identity of the coding system will usually be MC (Medicare provider number or HIBCC site codes) in the United States.  Each country may want to specify its preferred coding system and define a coding system ID to identify it.

Remember that the magnitude of a treatment or the setting on a machine, such as a ventilator, can be regarded as an observation.  Thus, pharmacy, respiratory care, and nursing may be producers of such observations.

#### Observation description  (TX)   00591

Definition:  This field contains a text description of this observation.

#### Other test/observation IDs for the observation  (CE)   00592

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains all alias codes/identifiers for this observation.  If more than one alias code needs to be specified, multiple three-component, CE-format entries (<code 1>^<name 1>^<code system 1>) may be given, separated by repeat delimiters.  An observation may have as many names/codes as are applicable (e.g., ICD9, ACR-NEMA, SNOMED, and READ).  We encourage the inclusion of as many different codes as may apply to assist cross-system mapping of terminology.  All components of each triplet should be non-null (that is, names and coding system IDs within the CE data type are required in addition to codes).  The source list may be any of those included in ASTM Tables 3 and 5.

Because the size (dose) of a treatment can also be an observation, codes that identify treatments (e.g., NDC, ICCS) may also be included in this field.

Note: In this field, the names within the CE data type are required.

#### Other names (recognized by the producer for the observation)  (ST)   00593

Definition:  This field contains any test aliases or synonyms for the name in the context of the ordering service.  These are alternative names, not associated with a particular coding system, by which the battery, test, or observation (e.g., measurement, test, diagnostic study, treatment) is known to users of the system.  Multiple names in this list are separated by repeat delimiters.

#### Preferred report name for the observation  (ST)   00594

Definition:  This field contains the preferred name for reporting the observation or battery.  The name can contain up to 30 characters (including blanks).  It is the preferred name for columnar reports that require a maximum name size.

#### Preferred short name or mnemonic for the observation  (ST)   00595

Definition:  This field contains the name that can be used in space-limited reports (e.g., specimen labels) to identify the observation for the convenience of human readers.  The name can contain up to eight characters.

#### Preferred long name for the observation  (ST)   00596

Definition:  This field contains the fully-specified name for the observation or battery.  It may include the full (unabbreviated) multiple-word names and contain up to 200 characters.  It should be as scientifically precise as possible.

#### Orderability  (ID)   00597

Definition:  This field indicates whether or not a test/observation is an orderable code.   Refer to HL7 table 0136 - Yes/no indicator for valid values.

Y 	the test/observation is an orderable code

N 	the test/observation is not orderable

For example, blood differential count is usually an orderable “test,” MCV, contained within the differential count, is usually not independently orderable.

#### Identity of Instrument used to perform this study  (CE)   00598

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition: When applicable, this field identifies the instrument or device that is used to generate this observation or battery.  Examples are the automated instrument in the laboratory, the imaging device and model number in radiology, and the automatic blood pressure machine on the ward.  The instrument is specified as a coded entry in anticipation that these identifiers could be specified as codes.  Initially, we expect that most of the information about devices will be transmitted as text in the second component of the CE identifier.  If more than one kind of instrument is used, all of them can be listed, separated by repeat delimiters.

#### Coded representation of method  (CE)   00599

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the method(s) used to produce the observation and should be recorded in a computer-understandable (coded) form here.  This field should report the same method(s) reported in narrative in the following field.  More than one method may be listed, but only if they produce results that are clinically indistinguishable.  Multiple methods must be separated by repeat delimiters.

#### Portable  (ID)   00600

Definition:  This field indicates whether or not a portable device may be used for the test/observation.  Refer to HL7 table 0136 - Yes/no indicator for valid values.

Y 	the observation can be obtained with a portable device brought to the patient

N 	the patient or specimen must be transported to the device

#### Observation producing department/section  (CE)   00601

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field permits the sorting of observation orders and values by the providing service’s department/section.  It provides “source oriented” reporting when required.  The codes for this field should be taken from ASTM Table 15 (Diagnostic Service Codes).  Free text may be used instead of these codes, but in that case, they should be recorded as the second “component” of the field to distinguish them from the standard codes.  Multiple codes in this field are separated by repeat delimiters.

#### Telephone number of section  (XTN)   00602

Definition:  This field contains the telephone number for calling responsible parties in this section to ask results or advice about the use of this test.

#### Nature of test/observation  (IS)   00603

Definition:  This field indicates whether the definition entry identifies a test battery, an entire functional procedure or study, a single test value (observation), multiple test batteries or functional procedures as an orderable unit (profile), or a single test value (observation) calculated from other independent observations.  Refer to user-defined table 0174 - Nature of test/observation for suggested values.

User-defined Table 0174 - Nature of test/observation

| - Value | - Description |
| --- | --- |
| P | Profile or battery consisting of many independent atomic observations (e.g., SMA12, electrolytes), usually done at one instrument on one specimen |
| F | Functional procedure that may consist of one or more interrelated measures (e.g., glucose tolerance test, creatine clearance), usually done at different times and/or on different specimens |
| A | Atomic test/observation (test code or treatment code) |
| S | Superset--a set of batteries or procedures ordered under a single code unit but processed as separate batteries (e.g., routines = CBC, UA, electrolytes) This set indicates that the code being described is used to order multiple test/observation batteries. For example, a client who routinely orders a CBC, a differential, and a thyroxine as an outpatient profile might use a single, special code to order all three test batteries, instead of having to submit three separate order codes. |
| C | Single observation calculated via a rule or formula from other independent observations (e.g., Alveolar--arterial ratio, cardiac output) |

Codes P, F, and S identify sets (batteries) and should be associated with an OM5 segment that defines the list of elements.  The definitions for the contained elements would have to be sent in other independent OMx segments, one for each contained element.  In the ASTM context, most text reports--such as discharge summaries, admission H&Ps, and chest X-ray reports--are considered as sets, in which each section of the report (e.g., description, impression, and recommendation of an X-ray report) is considered a separate observation.

Code A identifies a single direct observation and would usually be associated with an OM2 and/or OM3 segments.

Code C identifies a derived quantity and would usually be associated with an OM6 segment.

All of these codes can be associated with one or more OM4 (specimen) segments.

#### Report subheader  (CE)   00604

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains an optional string that defines the preferred header under which this observation should be listed on a standard display.  For example, if the test is hemoglobin, this string might be “Complete blood count.”  It is represented as a coded data type so that a battery can be a header.  Only the description part of the string may be included in case the subheader does not have an associated code.  When a series of observations is displayed according to the sort order given below, the subheader that groups those observations is presented whenever the subheader changes.

#### Report display order  (ST)   00605

Definition:  This field contains an optional string that defines the sort order in which this observation is presented in a standard report or display that contains the many observations.

#### Date/time stamp for any change in definition for the observation  (TS)   00606

Definition:  This field contains the date and time that the last of any field change was made and in the host’s record corresponding to the OM1 segment.

#### Effective date/time of change .  (TS)  00607

Definition:  This field contains the date and time of the last change in the test procedure that would make previous results incompatible with new results, e.g., the last time that normal reference range or units changed for a numeric test/observation.

We strongly suggest that observation producers never use the same observation ID when the measurement procedures change in such a way that results produced under the new procedure are clinically different from those produced with the old procedure. Rather, the producer should try to adjust the new procedure so that its values are clinically indistinguishable from the old. Failing that, one should create a new observation ID for the observation produced under the new procedure.

In the rare circumstances when a procedure change occurs and neither of the above two options is viable, this field shall be used to transmit the effective date/time of the new procedure. The receiving system shall assume that any values that come across under this observation ID are under the new procedure after this date and take appropriate steps to distinguish the old from the new observations.

This number is included to provide a means of communicating with the observation producing service when they have questions about particular observations or results.

#### Typical turn-around time (NM)   00608

Definition:  This field contains the typical processing time for single test/observation.  This field indicates the time from the delivery of a specimen or transport of a patient to a diagnostic service and the completion of the study.  It includes the usual waiting time.  The units are measured in minutes.

#### Processing time  (NM)   00609

Definition:  This field contains the usual length of time (in minutes) between the start of a test process and its completion.

#### Processing priority  (ID)   00610

Definition:  This field contains one or more available priorities for performing the observation or test.  This is the priority that can be placed in OBR-27-quantity/timing.  For tests that require a specimen, this field may contain two components in the format <specimen priority>^<processing priority>.  The first component in this case indicates the priority with which the specimen will be collected and is the priority that is specified in an OBR segment when ordering the observation.  The second component indicates the corresponding priority with which the producer service will process the specimen, produce the observation, and return results, when this differs from collection priority.  Refer to HL7 table 0168 - Processing priority for valid values.

Table 0168 - Processing priority

| Value | Description |
| --- | --- |
| S | Stat (do immediately) |
| A | As soon as possible (a priority lower than stat) |
| R | Routine |
| P | Preoperative (to be done prior to surgery) |
| T | Timing critical (do as near as possible to requested time) |
| C | Measure continuously (e.g., arterial line blood pressure) |
| B | Do at bedside or portable (may be used with other codes) |

The priority for obtaining the specimen is included in OM4.  Multiple priorities may be given, separated by repeat delimiters.  For example, S~A~R~P~T indicates that the test may be ordered using codes S, A, R, P, or T.

#### Reporting priority  (ID)   00611

Definition:  This field contains the available priorities reporting the test results when the user is asked to specify the reporting priority independent of the processing priority.  Refer to HL7 table 0169 - Reporting priority for valid values.

- Table 0169 - Reporting priority

| Value | Description |
| --- | --- |
| C | Call back results |
| R | Rush reporting |

#### Outside site(s) where observation may be performed  (CE)   00612

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the identification(s) of the outside service(s) that produce(s) the observation.  The format of this CE field uses the producer ID (as defined in OM1-5-producer ID) and the name of the service separated by component delimiters.  An example is 39221^ACME lab^MC.  If multiple services are used, they should be separated by repeat delimiter(s).

#### Address of outside site(s)  (XAD)   00613

Components:	<street address (ST)>  ^ <other designation (ST)> ^ <city (ST)>  ^ <state or province (ST)>  ^ <zip or postal code (ST)> ^ <country (ID)> ^ <address type> (ID)> ^ <other geographic designation (ST)> ^ <country/parish code (IS)> ^ <census tract (S)> ^ <address representation code (ID)>

Definition:  This field contains the address of the outside services listed in OM1-28-address of outside site(s) where observation may be performed.  If multiple services are recorded in that field, their addresses should be separated by repeat delimiters, and the addresses should appear in the same order in which the services appear in the preceding field.

#### Phone number of outside site  (XTN)   00614

Components:  [NNN] [(999)]999-9999 [X99999] [B99999] [C any text] ^ <telecommunication use code (ID)> ^ <telecommunication equipment type (ID)> ^ <email address (ST)> ^ <county code (NM)> ^ <area/city code (NM)> ^ <phone number (NM) ^ <extension (NM)> ^ <any text (ST)>

Definition:  This field contains the telephone number of the outside site.

#### Confidentiality code  (IS)   00615

Definition:  This field contains the degree to which special confidentiality protection should be applied to the observation.  For example, a tighter control may be applied to an HIV test than to a CBC.  Refer to user-defined table 0177 - Confidentiality code for suggested values.

User-defined Table 0177 - Confidentiality code

| Value | Description |
| --- | --- |
| V | Very restricted |
| R | Restricted |
| U | Usual control |
| EMP | Employee |
| UWM | Unwed mother |
| VIP | Very important person or celebrity |
| PSY | Psychiatric patient |
| AID | AIDS patient |
| HIV | HIV(+) patient |
| ETH | Alcohol/drug treatment patient |

#### Observations required to interpret this observation  (CE)   00616

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the list of variables that the diagnostic service needs to interpret the results of an ordered study.  The observations specified here should be sent to the diagnostic service as OBX segments along with the order (OBR) segment.

Example for cervical pap smear:

2000.32^date last menstrual period^AS4~2000.33^menstrual state^AS4

Example for arterial blood gas:

94700^inspired 02^AS4

These examples use AS4 codes in code/text format to identify the variables.  Separate multiple items by repeat delimiters.

#### Interpretation of observations  (TX)   00617

Definition:  This field contains the clinical information about interpreting test results.  Examples are the conditions (drugs) that may cause false abnormals, and the information about the sensitivity and specificity of the test for diagnoses.

#### Contraindications to observations  (CE)   00618

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the diagnosis or problem for which the test is a contraindication or of possible danger (e.g., pacemaker, pregnancy, diabetes).  For example, if the test identified in OM1 was an intravenous pyelogram, this field would include warnings about the use of contrast media in diabetes. The contraindication diagnoses should be separated by repeat delimiters.

Most contraindication rules will be transmitted as free text.  In such cases, the contents serve only as information for human reading.  However, an alternative for machine readable contraindication rules also exists. The rule may be defined formally in the Arden Syntax (ASTM 1460-1992) which has syntax for defining algebraic and transcendental equations, as well as temporal and logical selection criteria based on patient information stored in the computer record. Reflex rules that are written in Arden Syntax should begin and end with a double semi-colon (;;), the Arden slot delimiter.

#### Reflex tests/observations  (CE)   00619

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the test names as type CE (i.e., <code>^<text name>^<coding system>) that may be ordered automatically by the diagnostic service, depending on the results obtained from the ordered battery.  A screening CBC might trigger a reticulocyte count if the Hgb is less than 12.  Multiple reflex tests are separated by repeat delimiters.

#### Rules that trigger reflex testing  (TX)   00620

Definition:  This field contains the rules that trigger the reflex tests listed above.  If multiple reflex tests are listed in OM1-34-reflex tests/observations separated by repeat delimiters, a set of corresponding rules will be included in this section.  The first rule will apply to the first test, the second to the second test, and so on.

Most reflex rules will usually be transmitted as free text.  In such cases, the contents serve only as information for human reading.  However, an alternative for machine readable rules also exists. The rule may be defined formally in the Arden Syntax (ASTM 1460-1992) which has syntax for defining algebraic and transcendental equations, as well as temporal and logical selection criteria based on patient information stored in the computer record.  Reflex rules that are written in Arden Syntax should begin and end with a double semi-colon (;;), the Arden slot delimiter.

#### Fixed canned message  (CE)   00621

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the codes and a fixed text message that is always associated with an abbreviation.  The field may include multiple messages separated by repeat delimiters.

Most rules about patient testing will be transmitted as free text.  In such cases, the contents serves only as information for human reading.  However, an alternative for machine readable rules also exists.  The rule may be defined formally in the Arden Syntax (ASTM 1460-1992) which has syntax for defining algebraic and transcendental equations, as well as temporal and logical selection criteria based on patient information stored in the computer record.  Rules about patient preparation are written in Arden Syntax should begin and end with a double semi-colon (;;), the Arden slot delimiter.

#### Patient preparation  (TX)   00622

Definition:  This field contains the tests or observations that require special patient preparation, diet, or medications.  For GI contrast studies, this field would contain the pretest diet, e.g., low residue for two days, NPO before study, and the preferred purgatives.  Each separate med, diet, or preparation should be delimited by a repeat delimiter.  Separate each requirement by a repeat delimiter.  Example for a sigmoidectomy: clear liquid diet full day before procedure~take 8 oz mag citrate 6pm day before procedure~take 2 ducat tabs (5m) at 4pm day before procedure~NPO past midnight.

#### Procedure medication  (CE)   00623

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the treatments that may be needed as part of the procedure.  Examples are radioactive iodine for a thyroid screen, and methacholine for a methacholine spirometry challenge.  This field should be identified as a CE data type.

#### Factors that may effect the observation  (TX)   00624

Definition:  This field contains the text description of the foods, diagnoses, drugs, or other conditions that may influence the interpretation of the observation. Information about the direction of the effect, and any recommendation about altering the diet, conditions, or drug before initiating the test observation.

Most rules about factors that effect the test interpretation will be transmitted as free text.  In such cases, the contents serves only as information for human reading.  However, an alternative for machine readable rules also exists.  The rule may be defined formally in the Arden Syntax (ASTM 1460-1992) which has syntax for defining algebraic and transcendental equations, as well as temporal and logical selection criteria based on patient information stored in the computer record.  Rules about patient preparation are written in Arden Syntax and should begin and end with a double semi-colon (;;), the Arden slot delimiter.

#### Test/observation performance schedule  (ST)   00625

Definition:  This field contains the diagnostic studies/tests that are performed only at certain times during the course of a work day or work week.  This field indicates the maximum interval between successive test performances (the test may actually be performed more frequently).  The format given in Chapter 4, Section 4.4.2.1, “Repeat Pattern,” should be used.  If necessary, multiple codes may be given, separated by repeat delimiters.  The use of multiple codes indicates that the test is performed at multiple concurrent intervals.  For example, Q6H indicates that the test is performed at least once every 6 hours around the clock.  QJ1 indicates that the test is performed at least every week on Mondays.  QAM~QPM indicates that the test is performed at least once every morning and every evening. QJ1~QJ3~QJ5 indicates that the test is performed at least every week on Mondays, Wednesdays, and Fridays.  C indicates that the test is performed continuously, 7 days per week.

#### Description of test methods (TX)   00626

Definition:  This field contains the text description of the methods used to perform the text and generate the observations.  Bibliographic citations may be included.

#### Kind of quantity observed  (CE)   00937

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definitions:  This optional attribute describes the underlying kind of property represented by this observation.  This attribute distinguishes concentrations from total amounts, molar concentrations from mass concentrations, partial pressures from colors, and so forth.  These are discussed more fully in the LOINC Users’ Manual.1  They are derived from the approach described in 1995 edition of the IUPAC Silver Book. These distinctions are used in IUPAC and LOINC standard codes.  Defined categories are listed in HL7 table 0254 - Kind of quantity.

The distinctions of true quantities in this table are based primarily on dimensional analyses. The table contains a number of “families,” those related to simple counts (number, number concentration, etc.), to mass (mass, mass concentration, etc.), to enzyme activity (catalytic content, catalytic concentration, etc.), and molar or equivalents (substance content, substance concentration).

By this classification, a glucose (in the US) would be classed as a mass concentration. A sodium would be classed as a substance concentration. Within the family, a total amount should be described as the unadorned variant; e.g., the property of measure for a patient’s weight would be mass, not mass content.  Most chemical measures produce concentrations, as exemplified by sodium and glucose. However, a 24-hour urine protein is not a mass concentration, but a mass rate (mass per unit time).  The content variants (e.g., mass content, substance content) are used to reflect an amount per mass (usually) of tissue.

This attribute would be valued in a master file only if the service sending the master file classified observations by their principle of measurement.

Table 0254 - Kind of quantity

| Enzymatic Activity |  |  | Other Properties |  |  |
| --- | --- | --- | --- | --- | --- |
| CACT | *Catalytic Activity | ABS | Absorbance |
| CNC | *Catalytic Concentration | ACT | *Activity |
| CCRTO | Catalytic Concentration Ratio | APER | Appearance |
| CCNT | *Catalytic Content | ARB | *Arbitrary |
| CFR | *Catalytic Fraction | AREA | Area |
| CRAT | *Catalytic Rate | ASPECT | Aspect |
| CRTO | Catalytic Ratio | CLAS | Class |
| Entitic |  |  | CNST | *Constant |
| ENT | *Entitic | COEF | *Coefficient |
| ENTSUB | *Entitic Substance of Amount | COLOR | Color |
| ENTCAT | *Entitic Catalytic Activity | CONS | Consistency |
| ENTNUM | *Entitic Number | DEN | Density |
| ENTVOL | *Entitic Volume | DEV | Device |
| Mass |  |  | DIFF | *Difference |
| MASS | *Mass | ELAS | Elasticity |
| MCNC | *Mass Concentration | ELPOT | Electrical Potential (Voltage) |
| MCRTO | *Mass Concentration Ratio | ELRAT | Electrical current (amperage) |
| MCNT | Mass Content | ELRES | Electrical Resistance |
| MFR | *Mass Fraction | ENGR | Energy |
| MINC | *Mass Increment | EQL | Equilibrium |
| MRAT | *Mass Rate | FORCE | Mechanical force |
| MRTO | *Mass Ratio | FREQ | Frequency |
| Counts |  |  | IMP | Impression/ interpretation of study |
| NUM | *Number | KINV | *Kinematic Viscosity |
| NCNC | *Number Concentration | LEN | Length |
| NCNT | *Number Content | LINC | *Length Increment |
| NFR | *Number Fraction | LIQ | *Liquifaction |
| NRTO | *Number Ratio | MGFLUX | Magnetic flux |
| Substance Amount (Moles/Millequivalents) |  |  | MORPH | Morphology |
| SUB | *Substance Amount | MOTIL | Motility |
| SCNC | *Substance Concentration | OD | Optical density |
| SCRTO | *Substance Concentration Ratio | OSMOL | *Osmolality |
| SCNT | *Substance Content | PRID | Presence/Identity/Existence |
| SCNTR | *Substance Content Rate | PRES | *Pressure (Partial) |
| SFR | *Substance Fraction | PWR | Power (wattage) |
| SCNCIN | *Substance Concentration Increment | RANGE | *Ranges |
| SRAT | *Substance Rate | RATIO | *Ratios |
| SRTO | *Substance Ratio | RDEN | *Relative Density |
| Volumes |  |  | REL | *Relative |
| VOL | *Volume | SATFR | *Saturation Fraction |
| VCNT | *Volume Content | SHAPE | Shape |
| VFR | *Volume Fraction | SMELL | Smell |
| VRAT | *Volume Rate | SUSC | *Susceptibility |
| VRTO | *Volume Ratio | TASTE | Taste |
| Miscellaneous Unit Measures |  |  | TEMP | *Temperature |
| ACNC | Concentration, Arbitrary Substance | TEMPDF | *Temperature Difference |
| RLMCNC | *Relative Mass Concentration | TEMPIN | *Temperature Increment |
| RLSCNC | *Relative Substance Concentration | TITR | *Dilution Factor (Titer) |
| THRMCNC | *Threshold Mass Concentration | TYPE | *Type |
| THRSCNC | *Threshold Substance Concentration | VEL | *Velocity |
|  | Time | VELRT | *Velocity Ratio |
| TIME | *Time (e.g. seconds) | VISC | *Viscosity |
| TMDF | *Time Difference |  |  |
| TMSTP | *Time Stamp -- Date and Time |  |  |
| TRTO | *Time Ratio |  |  |
| RCRLTM | *Reciprocal Relative Time |  |  |
| RLTM | *Relative Time |  |  |
| *Starred items are adopted from the IUPAC Silver Book,2 non-starred items are extensions. |  |  |  |  |  |  |

#### Point versus interval  (CE)   00938

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This optional attribute allows master files to classify observations as measuring the patient’s state at a point in time (e.g., spot urines, random urines, serum potassium), or averaged over a interval of time (e.g., concentration, total amount, or clearance over a 24-hour collection).  Interval measures most often apply to urine and stool specimens (e.g., 24-hour urines, 3-day stool fats).  They also apply to clinical measurements such as urine outputs, which are reported as shift totals and 24-hour totals, and event counts on physiologic monitors such as the number of PVCs on a 24-hour Holter monitor.

This field would only be valued in a transaction if the service sending this master file message classified its observation by point versus time interval. This field is not used to record the time collection interval for a particular sample. It is used to specify a characteristic of an observation which has a defined normal range and to distinguish observations of the same kind but observed over varying periods of time.  A spot urine sodium would have PT stored in this field. A 24-hour urine sodium and a 24-hour Holter monitor would have 24H stored here. This attribute would only be valued if the filling service classified its observations by timing.  Refer to user-defined table 0255 - Duration categories for suggested values.

User-defined Table 0255 - Duration categories

| PT | To identify measures at a point in time. This is a synonym for "spot" or "random" as applied to urine measurements. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| * (star) | Life of the "unit." Used for blood products. |  |  |  |  |  |  |  |  |
| 30M | 30 minutes | 7H | 7 hours | 6D | 6 days |
| 1H | 1 hour | 8H | 8 hours | 1W | 1 week |
| 2H | 2 hours | 12H | 12 hours | 2W | 2 weeks |
| 2.5H | 2½ hours | 24H | 24 hours | 3W | 3 weeks |
| 3H | 3 hours | 2D | 2 days | 4W | 4 weeks |
| 4H | 4 hours | 3D | 3 days | 1L | 1 months (30 days) |
| 5H | 5 hours | 4D | 4 days | 2L | 2 months |
| 6H | 6 hours | 5D | 5 days | 3L | 3 months |

#### Challenge information  (TX)   00939

Definition:  This optional attribute provides information for classifying observations by the challenge component of the test, if a challenge does speciate the observation.  For example, distinguishing tests that have a challenge component in database. There co-ascribes the physiologic or drug challenge that is intrinsic to the measurement. To identify, for example, tests that include a glucose challenge.

To construct this text string, use the following template.  (Note:  This field is not constructed of formally defined components; it is a free text field.  Component delimiters are not used and it is not necessary to supply placeholders if some “components” are not used.)

The time delay follows the syntax: n<S|M|H|D|W> where  n is a number (possibly a decimal);   S denotes seconds;  M denotes minutes;  H denotes hours;  D denotes days; and W denotes weeks.  The time delay can be preceded by a ‘greater than’ (>) sign, e.g. >4H.

HL7 table 0256 - Time delay post challenge lists possible values for time delay.

Examples

PRE 100 GM GLUCOSE PO

PRE 100 GM GLUCOSE PO

30M POST 100 GM GLUCOSE PO

2H POST 100 GM GLUCOSE PO

TROUGH

For drug peak and trough measures the nature of the substance challenged is the same as the analyte name, and need not be included.

We denote the route of the challenge via abbreviations for medication routes (see Chapter 4, Section 4.8.3.1, “Route,” HL7 table 0162 - Route of administration).  An oral route of administration would be denoted by “PO,” an intravenous route by “IV.”

Details of the drug dose, time the dose was given, route of administration, etc., would be noted in separate OBX, and would have corresponding master observation definitions stored in the observation master file map to different records stored in the master file segments contained in the drug level message.

Table 0256 - Time delay post challenge

| BS Baseline (time just before the challenge) PEAK The time post drug dose at which the highest drug level is reached (differs by drug) TROUGH The time post drug dose at which the lowest drug level is reached (varies with drug) RANDOM Time from the challenge, or dose not specified. (random) |  |  |
| --- | --- | --- |
| n minutes/hours/days/weeks/months/etc. after challenge begun: |  |  |
| 1M 1 minute post challenge 2M 2 minutes post challenge 3M 3 minutes post challenge 4M 4 minutes post challenge 5M 5 minutes post challenge 6M 6 minutes post challenge 7M 7 minutes post challenge 8M 8 minutes post challenge 9M 9 minutes post challenge 10M 10 minutes post challenge 15M 15 minutes post challenge 20M 20 minutes post challenge 25M 25 minutes post challenge 20M 30 minutes post challenge 1H 1 hour post challenge 2H 2 hours post challenge 2.5H 2 1/2 hours post challenge 3H 3 hours post challenge 4H 4 hours post challenge 5H 5 hours post challenge | 6H 6 hours post challenge 7H 7 hours post challenge 8H 8 hours post challenge 8H SHIFT 8 hours aligned on nursing shifts 12H 12 hours post challenge 24H 24 hours post challenge 2D 2 days 3D 3 days 4D 4 days 5D 5 days 6D 6 days 7D 7 days 1W 1 week 10D 10 days 2W 2 weeks 3W 3 weeks 4W 4 weeks 1L 1 month (30 days) post challenge 2L 2 months (60 days) post challenge 3L 3 months (90 days) post challenge |

The nature of a physiologic (non-drug) challenge may also be specified, using the terms in HL7 table 0257 - Nature of challenge.

Table 0257 - Nature of challenge

| Value | Description |
| --- | --- |
| CFST | Fasting (no calorie intake) for the period specified in the time component of the term, e.g., 1H POST CFST |
| EXCZ | Exercise undertaken as challenge (can be quantified) |
| FFST | No fluid intake for the period specified in the time component of the term |

#### Relationship modifier (CE)   00940

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This optional attribute provides a mechanism for classifying observations according to the subject, in relation to the patient whose results might be stored with as “patient” data.  It is standard practice, for example, to report values for controls, donors, and blood product units as well as the patient’s own values, and store them in the patient’s record. (This may not be the best way to model such information, but it is the way it is usually reported.) This should be valued when two values (e.g., one for patient and one for a blood product unit) could otherwise be confused.

The default value is “Patient,” and if not specified, this value is assumed.  The persons sub-component can refer to HL7 table 0258 - Relationship modifier for valid values.

Table 0258 - Relationship modifier

| Value | Description |
| --- | --- |
| CONTROL | Control |
| PATIENT | Patient |
| DONOR | Donor |
| BPU | Blood product unit |

#### Target anatomic site of test  (CE)   00941

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This optional attribute formally indicates the site of the observation (to make it easy for a system to find all tests related to one anatomic site). It can be used to classify the observation by target site of the examination.  For example, “heart” might be recorded as the target of the electrocardiogram, cardiac echo, and thallium exercise test.  This attribute would be applicable to most imaging and electro-physiologic examinations. The SNOMED topology axis is an example of a coding system for anatomic sites.  User-defined tables may also apply here.

#### Modality of imaging measurement  (CE)   00942

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This optional attribute describes the modality used to classify the observations, e.g., radiograph, ultrasound, CT scan, NMR, etc..  This attribute is especially important for imaging studies.  Refer to user-defined table 0259 - Modality for suggested values; it is adopted from DICOM C.7.3.1.1.1 Modality.  If these are used, the code source ID would be DCM.

User-defined Table 0259 - Modality

| Value | Description | Value | Description |
| --- | --- | --- | --- |
| AS | Angioscopy | FS | Fundoscopy |
| BS | Biomagnetic Imaging | LP | Laparoscopy |
| CD | Color Flow Doppler | LS | Laser Surface Scan |
| CP | Colposcopy | MA | Magnetic Resonance Angiography |
| CR | Computed Radiography | MS | Magnetic Resonance Spectroscopy |
| CS | Cystoscopy | NM | Nuclear Medicine (radioisotope study) |
| CT | Computed Tomography | OT | Other |
| DD | Duplex Doppler | PT | Positron Emission Tomography (PET) |
| DG | Diapanography | RF | Radio Fluoroscopy |
| DM | Digital Microscopy | ST | Single Photon Emission Computed Tomography (SPECT) |
| EC | Echocardiography | TG | Thermography |
| ES | Endoscopy | US | Ultrasound |
| FA | Fluorescein Angiography | XA | X-ray Angiography |

### OM2 - numeric observation segment

This segment contains the attributes of observations with continuous values (including those with data types of numeric, date, or time stamp).  It can be applied to observation batteries of type A and C (see OM1-18-nature of test/observation).

Figure 8-8.  OM2 attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 | NM | O |  |  | 00586 | Sequence Number – Test/Observation Master File |
|  | 60 | CE | O |  |  | 00627 | Units of Measure |
|  | 10 | NM | O | Y |  | 00628 | Range of Decimal Precision |
|  | 60 | CE | O |  |  | 00629 | Corresponding SI Units of Measure |
|  | 60 | TX | O |  |  | 00630 | SI Conversion Factor |
|  | 200 | CM | O |  |  | 00631 | Reference (Normal) Range - Ordinal & Continuous Obs |
|  | 200 | CM | O |  |  | 00632 | Critical Range for Ordinal & Continuous Obs |
|  | 200 | CM | O |  |  | 00633 | Absolute Range for Ordinal & Continuous Obs |
|  | 200 | CM | O | Y |  | 00634 | Delta Check Criteria |
|  | 20 | NM | O |  |  | 00635 | Minimum Meaningful Increments |

- 8.7.4.0  OM2 field definitions

#### Sequence number – test/observation master file (NM)   00586

Definition:  This field contains the same value as the sequence number of the associated OM1 segment.

#### Units of measure  (CE)   00627

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the single tests/observations (those with a nature code of A or C, as described in OM1-18-nature of test/observation) that have numeric values.  This field contains their customary units of measure.

#### Range of decimal precision  (NM)   00628

Definition:  This field contains the numerically valued single observations (code A or C as described in OM1-18-nature of test/observation), specifies the total length in characters of the field needed to display the observation, and the number of digits displayed to the right of the decimal point.  This is coded as a single number in the format <length>.<decimal-digits>.  For example, a value of 6.2 implies 6 characters total (including the sign and decimal point) with 2 digits after the decimal point.  For integer values, the period and <decimal-digits> portion may be omitted (that is, 5.0 and 5 are equivalent).  More than one such mask may be transmitted (separated by repeat delimiters) when it is necessary to define multiple display formats that are possible.

#### Corresponding SI units of measure  (CE)   00629

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the single tests/observations - the corresponding SI units of measure in the format, when these differ from the customary units of measure given in the previous field.

#### SI conversion factor  (TX)   00630

Definition:  This field contains the continuous, numerically valued tests/observations, with a nature code of A or C (see OM1-18-nature of test/observation).  This is a factor for converting the customary units to SI units.

In the case that the observation units are not SI units, this field provides the formula needed to convert from the reported units to SI units, this shall include the equation needed to convert from the reporting to the SI units.

In the case that the relation is simply multiplicative, this field shall include only the conversion factor.  For example., if (results SI units) = c * (results reporting units),then only c would be stored in this field.  In the case of any other functional relationship, the entire equation would be stored as a test.

#### Reference (normal) range for ordinal and continuous observations  (CM)   00631

Definition:  This field contains the reference (normal) ranges for “numeric” observations/tests with a nature code of A or C (see OM1-18-nature of test/observation). It can identify different reference (normal) ranges for different categories of patients according to age, sex, race, and other conditions.

The general format is

- <ref. (normal) range1>^<sex1>^<age range1>^<age gestation1>^<species1>^<race/subspecies1>^<text condition1>~

- <ref. (normal) range2>^<sex2>^<age range2>^<age gestation2>^<species2>^<race/subspecies2>^<text condition2>~

- ·

- ·

- ·

- <ref. (normal) rangen>^<sexn>^<age rangen>^<age gestationn>^<speciesn>^<race/subspeciesn>^<text conditionn>

The components are defined in the following sections.

##### The reference (normal) range  (CM)

Components: <low value & high value>

Definition:  This subcomponent contains the reference (:normal) range.  The format of this field is where the range is taken to be inclusive (i.e., the range includes the end points).  In this specification, the units are assumed to be identical to the reporting units given in OM2-2-units of measure).

##### Sex  (IS)

Definition:  This subcomponent contains the sex of the patient.  Refer to user-defined table 0001 - Sex for suggested values.

##### Age range  (CM)

Subcomponents:  <low value & high value>

Definition:  This component contains the age range (in years or fractions thereof) specified as two values separated by a subcomponent delimiter (in order to allow a simple and consistent machine interpretation of this component).  Ages of less than one year should be specified as a fraction (e.g., 1 month = 0.0830, 1 week = 0.01920, 1 day = 0.0027300).  However, for most purposes involving infants, the gestational age (measured in weeks) is preferred.  The lower end of the range is not indicated; the upper end is, assuring that series of ranges do not overlap.

##### Gestational age range  (CM)

Subcomponents:  <low value & high value>

Definition:  This component contains the gestational age and is relevant only when the reference range is influenced by the stage of pregnancy.  A range of values is required.  The gestational age is measured in weeks from conception.  For example, <1&10> implies that the normals apply to gestational ages from 1 week to 4 weeks inclusive (1&4).  The lower end of the range is not included; the upper end is, assuring  that series of age ranges do not overlap.

##### Species  (TX)

Definition:  This component is assumed to be human unless otherwise stated.  The species should be represented as text (e.g., rabbit, mouse, rat).

##### Race/subspecies   (ST)

Definition:  In the case of humans (the default), the race is specified when race influences the reference range.  When normal ranges for animals are being described, this component can be used to describe subspecies or special breeds of animals.

##### Conditions  (TX)

Definition:  This component contains the condition as simply free text.  This component allows for definition of normal ranges based on any arbitrary condition, e.g., phase of menstrual cycle or dose of a particular drug.  It is provided as a way to communicate the normal ranges for special conditions.  It does not allow automatic checking of these text conditions.

##### Examples

A range that applies unconditionally, such as albumin, is transmitted as:

3.0 & 5.5

A normal range that depends on sex, such as Hgb, is transmitted as:

13.5 & 18^M~

12.0 & 16^F

A normal range that depends on age, sex, and race (a concocted example) is:

10 & 13   ^M^0  &  2 ^^^B

11 & 13.5 ^M^2  & 20 ^^^B~

12 & 14.5 ^M^20 & 70 ^^^B~

13 & 16.0 ^M^70 &    ^^^B

When no value is specified for a particular component, the range given applies to all categories of that component.  For example, when nothing is specified for race/species, the range should be taken as the human range without regard to race.  If no age range is specified, the normal range given is assumed to apply to all ages.  If the upper or lower end of a range is left out, it is assumed to be +infinity or -infinity, respectively.

When two different methods result in two different reference ranges, two different observations and corresponding OMx segments should be defined.

#### Critical range for ordinal and continuous observations  (CM)   00632

Components: <low value ^ high value>

Definition:  This field applies only to single tests/observations (i.e., a nature code of A or C, as described in OM1-18-nature of test/observation) with numeric results.  When a critical range is defined for such observations, it should be recorded here in the same format as the normal range (see OM2-6-reference (normal) range-ordinal & continuous obs).

#### Absolute range for ordinal and continuous observations  (CM)   00633

Components:  <range> ^ <numeric change> ^ <%/a change> ^ <days>

Definition:  This field applies only to single tests/observations with a nature code of A or C (see OM1-18-nature of test/observation).  It defines the range of possible results.  Results outside this range are not possible.  The field should be recorded in the same format as the normal and critical ranges.

#### Delta check criteria  (CM)   00634

Components:	<low & high (CM)> ^ <numeric threshold (NM)> ^ <change (ST)> ^ <length of time-days (NM)>

Definition:  This field applies to numeric tests/observations with a nature code of A or C (see OM1-18-nature of test/observation).  The field describes the information that controls delta check warnings and includes four components.

1)	The range to which the following applies: <low & high>.

All the ranges are defined in terms of the customary reporting units given in OM2-2-units of measure.  If no value range is given, the check applies to all values.

2)	The numeric threshold of the change that is detected, e.g., 10.

3)	Whether the change is computed as a percent change or an absolute change.  This component can have two possible values:

%	Indicates a percent change

a		Absolute change

4)	The length of time that the service retains a value for computing delta checks.  This is recorded in number of days.

More than one delta check rule can apply.  13&16^10^%^100~16.1&20^2^a^100 implies that the delta check will trigger on a 10% change when the value of the observation is between 13 and 16.  The check will trigger on an absolute change of 2 when the value is between 16.1 and 20.  In both cases, the system will keep the last result for 100 days.  In this example, beyond 100 days, the computer will not compute a delta check because it will not have a comparison value.

#### Minimum meaningful increments  (NM)   00635

Definition:  This field contains the numerically valued single observations (a nature code of A or C, as described in OM1-18-nature of test/observation) and specifies the smallest meaningful difference between reported values (the effective resolution of the measuring instrument or technique for continuous data, or the smallest discrete interval that can occur for discrete data).

### OM3 - categorical test/observation segment

This segment applies to free text and other non-numeric data types.

Figure 8-9.  OM3 attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 | NM | O |  |  | 00586 | Sequence Number – Test/Observation Master File |
|  | 60 | CE | O |  |  | 00636 | Preferred Coding System |
|  | 60 | CE | O |  |  | 00637 | Valid Coded “Answers” |
|  | 200 | CE | O | Y |  | 00638 | Normal Text/Codes for Categorical Observations |
| 5 | 200 | CE | O |  |  | 00639 | Abnormal Text/Codes for Categorical Observations |
| 6 | 200 | CE | O |  |  | 00640 | Critical Text Codes for Categorical Observations |
| 7 | 3 | ID | O |  | 0125 | 00570 | Value Type |

- 8.7.5.0	OM3 field definitions

#### Sequence number – test/observation master file (NM)   00586

Definition:  This field contains the same value as the sequence number of the associated OM1 segment.

#### Preferred coding system  (CE)   00636

Components:	<identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the observations whose categorical responses are taken from a specified table of codes (e.g., CE data types).  Record the preferred coding system for this observation (e.g., ICD9, SNOMED III).  Take the codes from ASTM Table 3 or 5, or specify a local code.

#### Valid coded “answers”  (CE)   00637

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains a list of valid coded answers.  In the case that the list of coded answers is easily enumerated, list the valid coded answers for this observation here using the preferred coding system given in OM3-2-preferred coding system.  If, for example, the given observation was VDRL, the valid answers might be non-reactive, 86^ intermediate, and 87^ reactive.

#### Normal text/codes for categorical observations  (CE)   00638

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  Certain observations/tests with a nature code of A or C (see OM1-18-nature of test/observation) have text (alpha) results (e.g., reactive, nonreactive).  Alpha normals for those tests should be entered in this field (e.g., “nonreactive”).

The format of this field is:

The first component is a code taken from a standard code source list.  The second component is the text associated with the code.  The third component is the identification of the code table source.  When only a text description of a possible answer is available, it is recorded as ^<text>.

Care should be taken to transmit only those results that are considered normal for that test.  A drug screen may have possible results of “negative” and “positive.”  However, only a result of “negative” is considered to be normal.  When an observation has more than one “normal” result, multiple values in this field should be separated with a repeat delimiter.

#### Abnormal text/codes for categorical observations  (CE)   00639

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the list of the text answers that are abnormal for the test.

#### Critical text/codes for categorical observations  (CE)   00640

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the list of coded results that are critically abnormal for this observation.

#### Value type  (ID)   00570

Definition:  This field contains the allowed data type for a single categorical observation (code A or C in OM1-18-nature of observation).  Refer to HL7 table - 0125 - Value type for valid values.

### OM4 - observations that require specimens segment

This segment applies to observations/batteries that require a specimen for their performance.  When an observation or battery requires multiple specimens for their performance (e.g., creatinine clearance requires a 24-hour urine specimen and a serum specimen), multiple segments may be included, one for each specimen type.

Figure 8-10.  OM4 attributes

| SEQ | LEN | DT | OPT |  | RP/# |  | TBL# |  | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 | NM | O |  |  |  |  | 00586 |  | Sequence Number – Test/Observation Master File |
|  | 1 | ID | O |  |  | 0170 |  | 00642 |  | Derived Specimen |
|  | 60 | TX | O |  |  |  |  | 00643 |  | Container Description |
|  | 20 | NM | O |  |  |  |  | 00644 |  | Container Volume |
|  | 60 | CE | O |  |  |  |  | 00645 |  | Container Units |
|  | 60 | CE | O |  |  |  |  | 00646 |  | Specimen |
|  | 60 | CE | O |  |  |  |  | 00647 |  | Additive |
|  | 10K | TX | O |  |  |  |  | 00648 |  | Preparation |
|  | 10K | TX | O |  |  |  |  | 00649 |  | Special Handling Requirements |
|  | 20 | CQ | O |  |  |  |  | 00650 |  | Normal Collection Volume |
|  | 20 | CQ | O |  |  |  |  | 00651 |  | Minimum Collection Volume |
|  | 10K | TX | O |  |  |  |  | 00652 |  | Specimen Requirements |
|  | 1 | ID | O | Y |  | 0027 |  | 00653 |  | Specimen Priorities |
|  | 20 | CQ | O |  |  |  |  | 00654 |  | Specimen Retention Time |

- 8.7.6.0  OM4 field definitions

#### Sequence number – test/observation master file (NM)   00586

Definition:  This field contains the same value as the sequence number of the associated OM1 segment.

#### Derived specimen  (ID)   00642

Definition:  This field contains the codes that identify the parents and children for diagnostic studies -- especially in microbiology -- where the initial specimen (e.g., blood) is processed to produce results (e.g., the identity of the bacteria grown out of the culture).  The process also produces new “specimens” (e.g., pure culture of staphylococcus, and E. Coli), and these are studied by a second order process (bacterial sensitivities). The parents (e.g., blood culture) and children (e.g., penicillin MIC) are identified in such cases. The codes are the following:

Table 0170 - Derived specimen

| Value | Description |
| --- | --- |
| P | Parent Observation |
| C | Child Observation |
| N | Not Applicable |

#### Container description  (TX)   00643

Definition:  This field contains the physical appearance, including color of tube tops, shape, and material composition (e.g., red-top glass tube).  Note that the color is not necessarily a unique identifier of the additive and/or use of the tube.  This is especially true for black and some blue tube tops, as can be seen above.  Color is included here for user convenience.

#### Container volume  (NM)   00644

Definition:  This field indicates the capacity of the container.

#### Container units  (CE)   00645

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the units of measure of the container volume.  If the units are ISO+ units, they should be recorded as single case abbreviations.  If the units are ANS+ or L (local), the units and the source code table must be recorded, except that in this case, component delimiters should be replaced by subcomponent delimiters.  For example, 1 indicates liters, whereas pt&&ANS+ indicates pints (ANSI units).  The default unit is milliliters (ml), which should be assumed if no units are reported.

#### Specimen  (CE)   00646

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field reports the specimen as one of the specimen codes described in ASTM Table 14 of 1238-91.  If multiple kinds of specimen are associated with this observation (as in the case for a creatinine clearance), separate them with repeat delimiters.

#### Additive  (CE)   00647

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the codes that should be those provided by NCCLS.  The following list is not exhaustive; it includes only examples.

| NAME | NCCLS Code Color | DESCRIPTION |
| --- | --- | --- |
| (1) Lithium Heparin -- anticoagulant | LIH Green | Dry powder. 10 to 30 USP units per mL of blood |
| (2) Sodium Heparin -- anticoagulant | NAH Green | Dried solution. 10 to 30 U.S.P. units per mL of blood |
| (3) Ethylenediaminetetraacetic acid; dipotassium salt [EDTA(K2)] | K2E Lavender | Dry powder. 1.5 to 2.2 mg per mL of blood |
| (4) Ethylenediaminetetraacetic acid; tripotassium salt [EDTA (K3)] | K3E Lavender | Clear solution. 1.5 to 2.2 mg per mL of blood |
| (5) Ethylenediaminetetraacetic acid; disodium salt [EDTA (Na2)] | N2E Lavender |  |

#### Preparation  (TX)   00648

Definition:  This field contains the special processing that should be applied to the container, e.g., add acidifying tablets before sending.

#### Special handling requirements  (TX)   00649

Definition:  This field contains the special handling requirements here (e.g., ice specimen, deliver within two hours of obtaining).

#### Normal collection volume  (CQ)   00650

Components: <quantity> ^ <units>

Definition:  This field contains the normal specimen volume required by the lab.  This is the amount used by the normal methods and provides enough specimens to repeat the procedure at least once if needed.  The default unit is milliliters (ml).

#### Minimum collection volume  (CQ)   00651

Components: <quantity> ^ <units>

Definition:  This field contains the amount of specimen needed by the most specimen sparing method (e.g., using micro techniques).  The minimum amount allows for only one determination.  The default unit is milliliters (ml).

#### Specimen requirements  (TX)   00652

Definition:  This field contains the other requirements for specimen delivery and special handling (e.g., delivery within one hour, iced).

#### Specimen priorities  (ID)   00653

Definition:  This field contains the allowed priorities for obtaining the specimen.  Note that they may be different from the processing priorities given in OM1-25-processing priority.  When a test is requested, the specimen priority given in OBR-27-quantity/timing should be one of the priorities listed here.  Multiple priorities are separated by repeat delimiters.  Refer to HL7 table 0027 - Priority for valid values.

Table 0027 - Priority

| Value | Description |
| --- | --- |
| S | Stat (do immediately) |
| A | As soon as possible (a priority lower than stat) |
| R | Routine |
| P | Preoperative (to be done prior to surgery) |
| T | Timing critical (do as near as possible to requested time) |

#### Specimen retention time  (CQ)   00654

Components: <quantity> ^ <units>

Definition:  This field contains the usual time that a specimen for this observation is retained after the observation is completed, for the purpose of additional testing.  The first component is the duration, and the second component is an ISO time unit.

### OM5 - observation batteries (sets) segment

This segment contains the information about batteries and supersets (a nature code of F, P or S, as described in OM1-18-nature of test/observation).

Figure 8-11.  OM5 attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 | NM | O |  |  | 00586 | Sequence Number – Test/Observation Master File |
| 2 | 200 | CE | O | Y |  | 00655 | Test/Observations Included within an Ordered Test Battery |
| 3 | 200 | ST | O |  |  | 00656 | Observation ID Suffixes |

- 8.7.7.0  OM5 field definitions

#### Sequence number – test/observation master file (NM)   00586

Definition:  This field contains the same value as the sequence number of the associated OM1 segment.

#### Tests/observations included within an ordered test battery  (CE)   00655

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the codes and names of all tests/observations included within a single battery (nature code P, as described in OM1-18-nature of test/observation), a single functional procedure (nature code F), or a given superset (nature code S).  When a segment includes a list of component elements, the sending system should be sure that the segments defining all of the components are sent before the segment that references them.  An entry in this list can itself be a battery.

The individual test/observation IDs should be recorded as type CE, i.e., in the standard format for coded observation identifiers.  Multiple observations should be separated by repeat delimiters.

If the definition segment defined serum electrolytes, this field might look like the following:

84132^potassium^AS4~

84295^sodium^AS4~

82435^chloride^AS4~

82374^HCO3^^AS4~

For S (superset) parameters, this field contains the batteries that are included within the “super” battery.  For example, ROUTINES might be defined as:

402^Electrolytes~352^Urinalysis~432^CBC~520^SMA12

#### Observation ID suffixes  (ST)   00656

Definition:  This field contains the tests or procedures that produce a type which uses observation ID suffixes following the test/observation ID code.  This field lists the possible options.  The applicable three-character mnemonics given in ASTM Table 20 (or others appropriate to the application) are listed, separated by repeat delimiters.  For example, a chest X-ray may use the suffixes IMP, REC, DEV, or others. Each of the expected suffixes should be listed here.

### OM6 - Observations that are calculated from other observations segment

This segment contains the information about quantities that are derived from one or more other quantities or direct observations by mathematical or logical means.

Figure 8-12.  OM6 attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 | NM | O |  |  | 00586 | Sequence Number |
|  | 10K | TX | O |  |  | 00657 | Derivation Rule |

- 8.7.8.0 OM6 field definitions

#### Sequence number  (NM)   00586

Definition:  This field contains the same value as the sequence number of the associated OM1 segment.

#### Derivation rule  (TX)   00657

Definition:  This field is used when there are patient variables that are derived from one or more other patient variables (e.g., creatinine clearance, ideal weight, maximum daily temperature, average glucose, framingham risk).  This field contains the rules for deriving the value of this variable (i.e., nature code C, as given in OM1-18-nature of test/observation).  These can be described in terms of humanly understandable formulas or descriptions.

When possible, however, they should be defined in terms of the Arden Syntax for specifying selection and transcendative functions and algebraic operations, ASTM E1460-92.  Derivation rules that are represented in Arden Syntax should begin and end with an Arden slot delimiter (;;).  Within this syntax, variables should be identified by OM1-2-producer's test/observation ID.  We recommend the use of the Arden Syntax because it permits the unambiguous specification of most such derived values and is a published standard for medical logic modules.

## LOCATION MASTER FILES

### Patient location master file message (MFN/MFK)

This section is specifically concerned with describing a master file message that should be used to transmit information which identifies the inventory of healthcare patient locations, such as nursing units, rooms, beds, clinics, exam rooms, etc.  In a network environment, this segment can be used to define patient locations to other applications.  The segment also includes the readiness states and support locations for the patient locations.

The LOC, LCH, LRL, LDP, and LCC segments must be preceded by the MFI and MFE segments, as described in Sections 8.8.2, “LOC - location identification segment,” through 8.8.68.3.”  In the following message, the MFI-1-master file identifier field should equal “LOC”

MFN					Master File Notification	Chapter

MSH			Message Header	2

MFI			Master File Identification	8

{MFE			Master File Entry	8

LOC			Patient Location Master	8

[{LCH}]		Location Characteristic	8

[{LRL}]		Location Relationship	8

{ LDP			Location Department	8

[{LCH}]		Location Characteristic	8

[{LCC}]		Location Charge Code	8

}

}

When the LCH segment appears immediately following the LOC segment, it communicates characteristics which are the same across multiple departments that may use the same room.  When the LCH segment appears immediately following the LDP segment, it communicates characteristics which differ for different departments that may use the same room.

MFK					Master File Acknowledgment	Chapter

MSH				Message Header	2

MSA				Acknowledgment	2

MFI				Master File Identification	8

[{MFA}]		Master File ACK 	8

Master Files Query Response:  When the LOC segment is used in the MFR message, the part of the message represented by:

{MFE

[Z..]}

is replaced by:

{MFE				Master File Entry

LOC 			Patient Location Master

[{LCH}]		Location Characteristic

[{LRL}]		Location Relationship

{LDP 			Location Department

[{LCH}]		Location Characteristic

[{LCC}]     }}Location Charge Code

### LOC - location identification segment

The LOC segment can identify any patient location referenced by information systems.  This segment gives physical set up information about the location.  This is not intended to include any current occupant or current use information.  There should be one LOC segment for each patient location.  If desired, there can also be one LOC segment for each nursing unit and room.

Figure 8-13.  LOC attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 200 | PL | R |  |  | 01307 | Primary Key Value - LOC |
| 2 | 48 | ST | O |  |  | 00944 | Location Description |
| 3 | 2 | IS | R | Y | 0260 | 00945 | Location Type - LOC |
| 4 | 90 | XON | O | Y |  | 00947 | Organization Name - LOC |
| 5 | 106 | XAD | O | Y |  | 00948 | Location Address |
| 6 | 40 | XTN | O | Y |  | 00949 | Location Phone |
| 7 | 60 | CE | O | Y |  | 00951 | License Number |
| 8 | 3 | IS | O | Y | 0261 | 00953 | Location Equipment |

8.8.2.0 	 LOC field definitions

#### Primary key value - LOC  (PL)   01307

Components:  <point of care (IS)> ^ <room (IS)> ^ <bed (IS)> ^ <facility (HD)> ^ <location status  (IS)> ^ <person location type (IS)> ^ <building (IS)> ^ <floor (IS)> ^ <location description (ST)>

Subcomponents of facility:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Definition:  This field contains the institution’s identification code for the location.  The identifying key value.   Must match MFE-4-primary key value.  This field has the same components as the patient location fields in the PV1 segment (except that bed status is not included here).

At least the first component of this field is required.  The first component can be an identifying code for the nursing station for inpatient locations, or clinic, department or home for patient locations other than inpatient ones.

#### Location description  (ST)   00944

Definition:  This field contains the optional free text description of the location, to elaborate upon LOC primary key value.

#### Location type - LOC (IS)   00945

Definition:  This field contains the code identifying what type of location this is.  Refer to user-defined table 0260 - Patient location type for suggested values.

User-defined Table 0260 - Patient location type

| Value | Description |
| --- | --- |
| N | Nursing Unit |
| R | Room |
| B | Bed |
| E | Exam Room |
| O | Operating Room |
| C | Clinic |
| D | Department |
| L | Other Location |

#### Organization name - LOC  (XON)   00947

Components:  <organization name (ST)> ^ <organization name type code (ID)> ^ <ID number (ID)> ^ <check digit (NM)> ^ < check digit scheme (ID)> ^ <assigning authority (HD)> ^ <identifier type code (ID)> ^ <assigning facility ID (HD)> ^ <name representation code)

Subcomponents of assigning authority:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Subcomponents of assigning facility:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Definition:  This field contains the organization of which this location is a part.  For inpatient locations, this can be the hospital or institution name.  For outpatient locations, this can be the clinic or office name.

#### Location address  (XAD)   00948

Components:	<street address (ST)> ^ <other designation (ST)> ^ <city (ST)> ^ <state or province (ST)> ^ <zip or postal code (ST)> ^ <country (ID)> ^ <address type (ID)> ^ <other geographic designation (ST)> ^ <county/parish code (IS)> ^ <census tract (IS)> ^ <address representation code (ID)>

Definition:  This field contains the address of the patient location, especially for use for outpatient clinic or office locations.

#### Location phone  (XTN)   00949

Components:	[NNN] [(999)]999-9999 [X99999] [B99999] [C any text] ^ <telecommunication use code (ID)> ^ <telecommunication equipment type (ID)> ^ <email address (ST)> ^ <county code (NM)> ^ <area/city code (NM)> ^ <phone number (NM) ^ <extension (NM)> ^ <any text (ST)>

Definition:  This field contains the phone number within the patient location, if any.  For example, the room or bed phone for use by the patient.

#### License number  (CE)   00951

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the multiple license numbers for the facility.

#### Location equipment  (IS)   00953

Definition:  This repeating field indicates what types of equipment are built in.  Applies only to room or bed locations.  If LOC-3-location type indicates that this is a room, this will be the equipment in the room which can be used by more than one bed.  If LOC-3-location type indicates this is a bed, this will be the bedside devices available to this bed.  Refer to user-defined table 0261 - Location equipment for suggested values.

User-defined Table 0261 - Location equipment

| Value | Description |
| --- | --- |
| OXY | Oxygen |
| SUC | Suction |
| VIT | Vital signs monitor |
| INF | Infusion pump |
| IVP | IV pump |
| EEG | Electro-Encephalogram |
| EKG | Electro-Cardiogram |
| VEN | Ventilator |

### LCH - location characteristic segment

The LCH segment is used to identify location characteristics which determine which patients will be assigned to the room or bed.  It contains the location characteristics of the room or bed identified in the preceding LOC segment. There should be one LCH segment for each attribute.

When the LCH segment appears immediately following the LOC segment, it communicates characteristics which are the same across multiple departments that may use the same room.  When the LCH segment appears immediately following the LDP segment, it communicates characteristics which differ for different departments that may use the same room.  For example, the following characteristics are more likely to vary by which department is using the room:  teaching, gender, staffed, set up, overflow, whereas the other characteristics are likely to remain the same.

Figure 8-14.  LCH attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 200 | PL | R |  |  | 01305 | Primary Key Value - LCH |
| 2 | 3 | ID | O |  | 0206 | 00763 | Segment Action Code |
| 3 | 80 | EI | O |  |  | 00764 | Segment Unique Key |
| 4 | 80 | CE | R |  | 0324 | 01295 | Location Characteristic ID |
| 5 | 80 | CE | R |  | 0136/ 0262/ 0263 | 01237 | Location Characteristic Value |

8.8.3.0 	 LCH field definitions

#### Primary key value - LCH (PL)   01305

Components:  <point of care (IS)> ^ <room (IS)> ^ <bed (IS)> ^ <facility (HD)> ^ <location status  (IS)> ^ <person location type (IS)> ^ <building (IS)> ^ <floor (IS)> ^ <location description (ST)>

Subcomponents of facility:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Definition:  This field contains the institution’s identification code for the location.  The identifying key value.  This field has the same components as the patient location fields in the PV1 segment (except that bed status is not included here).  At least the first component of this field is required.  The contents of this field must exactly match the content of its preceding MFE ( MFE-4-primary key value-MFE), its preceding LOC (LOC-1- primary key value-LOC), and its preceding LDP (LDP-1- primary key value-LDP).

#### Segment action code (ID)   00763

Definition:  This field indicates whether this repetition of the segment is being added, changed or deleted.  This repetition of the repeating segment must be identified using FT1-25-segment unique key.  The action code adds a validation check to indicate, from the point of view of the sending system, whether this repetition of a segment is being added, changed or deleted.  This and the following field are used to implement the “unique key” mode of updating repeating segments.  (See Chapter 2, Section 2.23.4.2, “Action code/unique identifier mode update definition.“.)  Refer to HL7 table 0206 - Segment action code for valid values.

#### Segment unique key (EI)   00764

Components:  <entity identifier (ST)> ^ <namespace ID (IS)> ^ <universal ID (ST)> ^ <universal ID type (ID)>

Definition:  This field contains a unique identifier for one of the multiple repetitions of this segment, to be used in conjunction with the preceding field.  Each of the repetitions of the segment will be uniquely identified by this unique key field for the purposes of updates.

#### Location characteristic ID  (CE)   01295

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains an identifier code to show WHICH characteristic is being communicated with this segment.  Refer to user-defined table 0324 - Location characteristic ID  for suggested values.

User-defined Table 0324 - Location characteristic ID

| Value | Description |
| --- | --- |
| SMK | Smoking |
| LIC | Licensed |
| IMP | Implant: can be used for radiation implant patients |
| SHA | Shadow: a temporary holding location that does not physically exist |
| INF | Infectious Disease: this location can be used for isolation |
| PRL | Privacy Level: indicating the level of private versus non-private room |
| LCR | Level of Care |
| OVR | Overflow |
| STF | Bed is Staffed |
| SET | Bed is Set Up |
| GEN | Gender of Patient(s) |
| TEA | Teaching Location |

#### Location characteristic value  (CE)   01237

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the value of the above field’s characteristic.  The expected coded values for this field will depend upon the previous field.  For example, if the previous field is SMK, IMP, INF, the values would be “Y” or “N”.

When LCH-4-location characteristic ID contains “SHA”- Shadow, refer to HL7 table 0136 - Yes/no indicator for valid values for LRL-5-organizational location relationship value.

Y	not a real bed, but a temporary holding location that does not physically exist in the
	census

N	this is a real bed

When LCH-4-location characteristic ID contains “PRL”- Privacy level  (CE) , then LRL-5- organizational location relationship value indicates how the room is set up and intended to be used, disregarding different uses under special circumstances.  Refer to user-defined table 0262 - Privacy level for suggested values.

User-defined Table 0262 - Privacy level

| Value | Description |
| --- | --- |
| F | Isolation |
| P | Private Room |
| J | Private Room - Medically Justified |
| Q | Private Room - Due To Overflow |
| S | Semi-Private Room |
| W | Ward |

When LCH-4-location characteristic ID contains “LCR”- Level of care, then LRL-5- organizational location relationship value contains the code which indicates what severity of the patient’s medical condition which this location is designed to handle.  This indicates how the room is set up and intended to be used, disregarding different uses under special circumstances.  Refer to user-defined table 0263 - Level of care.

User-defined Table 0263 - Level of care

| Value | Description |
| --- | --- |
| A | Ambulatory |
| E | Emergency |
| F | Isolation |
| N | Intensive Care |
| C | Critical Care |
| R | Routine |
| S | Surgery |

When LCH-4-location characteristic ID contains “IFD”- Infectious disease, refer to HL7 table 0136 - Yes/no indicator for valid values for LRL-5- organizational location relationship value.

Y	patients with infectious diseases can be admitted to this location, that is, this location can be used for isolation

N	this location cannot be used for isolation

When LCH-4-location characteristic ID contains “SMO”- Smoking, refer to HL7 table 0136 - Yes/no indicator for valid values for LRL-5- organizational location relationship value.

Y	this is a smoking location

N	this is a non-smoking location

When LCH-4-location characteristic ID contains “IMP”- Implant, refer to HL7 table 0136 - Yes/no indicator for valid values for LRL-5- organizational location relationship value.

Y	this location can be used by radiation implant patients

N	this location can not be used by radiation implant patients

When LCH-4-Location Characteristic ID contains “LIC”- Licensed, refer to HL7 table 0136 - Yes/no indicator for valid values for LRL-5- organizational location relationship value.

Y	this location is licensed

N	this location is not licensed

### LRL - location relationship segment

The LRL segment is used identify one location’s relationship to another location, the nearest lab, pharmacy, etc.

Figure 8-15.  LRL attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 200 | PL | R |  |  | 00943 | Primary Key Value - LRL |
| 2 | 3 | ID | O |  | 0206 | 00763 | Segment Action Code |
| 3 | 80 | EI | O |  |  | 00764 | Segment Unique Key |
| 4 | 80 | CE | R |  | 0325 | 01227 | Location Relationship ID |
| 5 | 80 | XON | C | Y |  | 01301 | Organizational Location Relationship Value |
| 6 | 80 | PL | C |  |  | 01292 | Patient Location Relationship Value |

8.8.4.0	LRL field definitions

#### Primary key value - LRL  (PL)   00943

Components:  <point of care (IS)> ^ <room (IS)> ^ <bed (IS)> ^ <facility (HD)> ^ <location status  (IS)> ^ <person location type (IS)> ^ <building (IS)> ^ <floor (IS)> ^ <location description (ST)>

Subcomponents of facility:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Definition:  This field contains the institution’s identification code for the location.  The identifying key value.  This field has the same components as the patient location fields in the PV1 segment (except that bed status is not included here).  At least the first component of this field is required.  The contents of this field must exactly match the content of its preceding MFE ( MFE-4-primary key value-MFE), its preceding LOC (LOC-1-primary key value-LOC), and its preceding LDP (LDP-1-primary key value-LDP).

#### Segment action code (ID)   00763

Definition:  This field indicates whether this repetition of the segment is being added, changed or deleted.  This repetition of the repeating segment must be identified using FT1-25-segment unique key.  The action code adds a validation check to indicate, from the point of view of the sending system, whether this repetition of a segment is being added, changed or deleted.  This and the following field are used to implement the “unique key” mode of updating repeating segments.  (See Chapter 2, Section 2.23.4.2, “Action code/unique identifier mode update definition.”)  Refer to HL7 table 0206 - Segment action code for valid values.

#### Segment unique key (EI)   00764

Components:  <entity identifier (ST)> ^ <namespace ID (IS)> ^ <universal ID (ST)> ^ <universal ID type (ID)>

Definition:  This field contains a  unique identifier for one of the multiple repetitions of this segment, to be used in conjunction with the preceding field.  Each of the repetitions of the segment will be uniquely identified by this unique key field for the purposes of updates.

#### Location relationship ID  (CE)   01227

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains an identifier code to show WHICH relationship is being communicated with this segment.  Refer to user-defined table 0325 - Location relationship ID  for suggested values.

User-defined Table 0325 - Location relationship ID

| Value | Description |
| --- | --- |
| RX | Nearest Pharmacy |
| RX2 | Second Pharmacy |
| LAB | Nearest Lab |
| LB2 | Second Lab |
| DTY | Nearest Dietary |
| ALI | Location Alias(es) |
| PAR | Parent Location |

#### Organizational location relationship value  (XON)  01301

Components:	<organization name (ST)> ^ <organization name type code (IS)> ^ <ID number (NM)> ^ <check digit (NM)> ^ <code identifying the check digit scheme employed (ID)> ^ <assigning authority (HD)> ^ <identifier type code (IS)> ^ <assigning facility ID (HD)> ^ <name representation code)

Subcomponents of assigning authority:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Subcomponents of assigning facility:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Definition:  This field is conditional on the value of LRL-4-location relationship ID.  When LRL-4-location relationship ID contains “RX”- Nearest Pharmacy, “RX2”- Other Pharmacy, “LAB”- Nearest Lab, “LB2”- Other Lab, or “DTY”- Dietary, this field holds that organization’s extended name i.e., the value of this field is conditional on the value of LRL-4-location relationship ID.  For example, for an inpatient location, this could be an in-house department ID code using only the third component of this data type.  For an outpatient location, this could be the nearest external pharmacy.

#### Patient location relationship value (PL) 01292

Components:  <point of care (IS)> ^ <room (ST)> ^ <bed (ST)> ^ <facility (HD)> ^ <status  (ID)> ^ <person location type (ID)> ^ <building (ID)> ^ <floor (ST)> ^ <location description (ST)>

Subcomponents of facility:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Definition:  This field is conditional on the value of LRL-4-location relationship ID.  When LRL-4-location relationship ID contains “ALI”- Location aliases or “PAR”- Parent location this field holds the value of the associated patient location.

When LRL-4-location relationship ID contains “PAR”- Parent, this field holds the value of the parent location to allow for nested entries. For example, a bed entry can point to its containing room or nurse unit.  The value for the parent location should match the  LOC-1-primary key value-LOC of the parent entry.  Not intended to be used for multiple designations of the same physical location, but for identifying the larger physical locations (supersets) which include this physical location as a subset.

### LDP - location department segment

The LDP segment identifies how a patient location room is being used by a certain department.  Multiple departments can use the same patient location, so there can be multiple LDP segments following an LOC segment.  There must be at least one LDP segment for each LOC segment.  This is not intended to include any current occupant information.

Figure 8-16.  LDP attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 200 | PL | R |  |  | 00963 | Primary Key Value - LDP |
| 2 | 10 | IS | R |  | 0264 | 00964 | Location Department |
| 3 | 3 | IS | O | Y | 0069 | 00965 | Location Service |
| 4 | 60 | CE | O | Y | 0265 | 00966 | Specialty Type |
| 5 | 1 | IS | O | Y | 0004 | 00967 | Valid Patient Classes |
| 6 | 1 | ID | O |  | 0183 | 00675 | Active/Inactive Flag |
| 7 | 26 | TS | O |  |  | 00969 | Activation Date LDP |
| 8 | 26 | TS | O |  |  | 00970 | Inactivation Date - LDP |
| 9 | 80 | ST | O |  |  | 00971 | Inactivated Reason |
| 10 | 80 | VH | O | Y | 0267 | 00976 | Visiting Hours |
| 11 | 40 | XTN | O |  |  | 00978 | Contact Phone |

8.8.5.0	LDP field definitions

#### Primary key value - LDP  (PL)   00963

Components:  <point of care (ID)> ^ <room (ST)> ^ <bed (ST)> ^ <facility (HD)> ^ <status  (ID)> ^ <person location type (IS)> ^ <building (ID)> ^ <floor (ST)> ^ <location description (ST)>

Subcomponents of facility:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Definition:  This field contains the institution’s identification code for the location.  The identifying key value.  This field has the same components as the patient location fields in the PV1 segment (except that bed status is not included here).  At least the first component of this field is required.  The contents of this field must exactly match the content of its preceding MFE (MFE-4-primary key value-MFE) and its preceding LOC (LOC-1-primary key value-LOC).

#### Location department  (IS)   00964

Definition:  This field contains the institution’s department to which this location belongs, or its cost center.  Refer to user-defined table 0264 - Location department for suggested values.

#### Location service  (IS)   00965

Definition:  This field contains the hospital or ancillary service with which this location is associated.  Depends on institution use.  Repeats for rooms that can be used, for example, by different services on different days.  These values should match the values used for PV1-10-hospital service, which is site defined.  Refer to user-defined table 0069 - Hospital service for suggested values.

#### Specialty type  (CE)   00966

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the specialty type (if any) of the department or clinic. This may also be considered a bed type.  Specialty type is a physical accommodation type, whereas ‘accommodation type’ (LCC-3-accommodation type) is a financial accommodation type.  Refer to user-defined table 0265 - Specialty type for suggested values.  See also LCH-4-location characteristic ID and LHC-5-Location Characteristic Value.

User-defined Table 0265 - Specialty type

| Value | Description |
| --- | --- |
| AMB | Ambulatory |
| PSY | Psychiatric |
| PPS | Pediatric Psychiatric |
| REH | Rehabilitation |
| PRE | Pediatric Rehabilitation |
| ISO | Isolation |
| OBG | Obstetrics, Gynecology |
| PIN | Pediatric/Neonatal Intensive Care |
| INT | Intensive Care |
| SUR | Surgery |
| PSI | Psychiatric Intensive Care |
| EDI | Education |
| CAR | Coronary/Cardiac Care |
| NBI | Newborn, Nursery, Infants |
| CCR | Critical Care |
| PED | Pediatrics |
| EMR | Emergency |
| OBS | Observation |
| WIC | Walk-In Clinic |
| PHY | General/Family Practice |
| ALC | Allergy |
| FPC | Family Planning |
| CHI | Chiropractic |
| CAN | Cancer |
| NAT | Naturopathic |
| OTH | Other Specialty |

#### Valid patient classes  (IS)   00967

Definition:  This field contains the patient types that are allowed to be assigned to this bed.  For example, Inpatient, Outpatient, Series, Clinic, ER, Ambulatory, Observation, etc.  These values should be the same set of values as those used for PV1-2-patient class.  Refer to user-defined table 0004 - Patient class for suggested values.

#### Active/inactive flag  (ID)   00675

Definition:  This field indicates whether the entry for this location is currently an active, that is, valid, usable entry (disregarding whether it’s waiting to be maintained by housekeeping).  Refer to HL7 table 0183 - Active/inactive for valid values.

#### Activation date - LDP  (TS)   00969

Definition:  This field contains the date and time when the location became active or “in service” for a department (disregarding whether it is waiting to be maintained by housekeeping).

#### Inactivation date - LDP  (TS)   00970

Definition:  This field contains the date when the location became inactive or “out of service” for this department (disregarding whether it is waiting to be maintained by housekeeping).

#### Inactivated reason  (ST)   00971

Definition:  This field contains the reason the location was put out of service.  It is used when LDP-8-inactivation date-LDP is sent.

#### Visiting hours  (VH)   00976

Components:  <start day range (ID)> ^ <end day range (ID)> ^ <start hour range (TM)> ^ <end hour range (TM)>

Definition:  This field contains the hours when this location is open for visiting.  Refer to HL7 table 0267 - Days of the week for valid values for the first two components.

Table  0267 - Days of the Week

| Value | Description |
| --- | --- |
| SAT | Saturday |
| SUN | Sunday |
| MON | Monday |
| TUE | Tuesday |
| WED | Wednesday |
| THU | Thursday |
| FRI | Friday |

#### Contact phone  (XTN)   00978

Components: [NNN] [(999)]999-9999 [X99999] [B99999] [C any text] ^ <telecommunication use code (ID)> ^ <telecommunication equipment type (ID)> ^ <email address (ST)> ^ <county code (NM)> ^ <area/city code (NM)> ^ <phone number (NM) ^ <extension (NM)> ^ <any text (ST)>

Definition:  This field contains the phone number to use to contact facility personnel about the patient location, in case of inquiries about the location.  This phone is not necessarily within the named patient location.

### LCC - location charge code segment

The optional LCC segment identifies how a patient location room can be billed by a certain department.  A department can use different charge codes for the same room or bed, so there can be multiple LCC segments following an LDP segment.

Figure 8-17.  LCC attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 200 | PL | R |  |  | 00979 | Primary Key Value - LCC |
| 2 | 10 | IS | R |  | 0264 | 00964 | Location Department |
| 3 | 60 | CE | O | Y | 0129 | 00980 | Accommodation Type |
| 4 | 60 | CE | R | Y | 0132 | 00981 | Charge Code |

8.8.6.0	LCC field definitions

#### Primary key value - LCC  (PL)   00979

Components:  <point of care (IS)> ^ <room (IS)> ^ <bed (IS)> ^ <facility (HD)> ^ <location status  (IS)> ^ <person location type (IS)> ^ <building (IS)> ^ <floor (IS)> ^ <location description (ST)>

Subcomponents of facility:  <namespace ID (IS)> & <universal ID (ST) & <universal ID Type (ID)

Definition:  This field contains the institution’s identification code for the location.  The identifying key value.  This field has the same components as the patient location fields in the PV1 segment (except that bed status is not included here).  At least the first component of this field is required.  The content of this field must exactly match the content of its preceding MFE ( MFE-4-primary key value-MFE), its preceding LOC (LOC-1-LOC primary key value), and its preceding LDP (LDP-1-primary key value-LDP).

#### Location department  (IS)   00964

Definition:  This field contains the institution’s department to which this location belongs, or its cost center.  It must match the value in its preceding LDP (LDP-2-location department).  Refer to user-defined table 0264 -Location department for suggested values.

#### Accommodation type  (CE)   00980

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the financial accommodation type of the bed or room which implies the rate to be used when occupied by a patient under specific medical conditions, which determines how it is billed.  Not the same as specialty type.  Used for general ledger categories.  Specialty type is a physical accommodation type, whereas this field is a financial accommodation type.  Repeating coded value.   Site-defined codes. Refer to user-defined table 0129 - Accommodation code for suggested values.

#### Charge code  (CE)   00981

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the repeating coded entry for codes identifying how the use of this location is to be charged.  For cross-referencing beds master files with the charge master files, or for generating charges when a patient is assigned to a bed.  These should be the same set of values used in FT1-7-transaction code.  Values are site negotiated.  Refer to user-defined table 0132 - Transaction code for suggested values.

### Example: MFN location master file message

MSH|^~\&|HL7REG|UH|HL7LAB|CH|19910918060544||MFN^M05|MSGID002|P|2.2||AL|NE<cr>

MFI|LOC||UPD|||AL<cr>

MFE|MAD|PMF98123789182|199110011230|3A^RM17^17-2^FAC1<cr>

LOC|3A^RM17^17-2^FAC1|BEST BED IN UNIT|B|UNIVERSITY HOSPITAL|54326 SAND P0INT WAY^^SEATTLE^WA^98199|(206)689-1329|92837465998|OXY<cr>

LCH|3A^RM17^17-2^FAC1|||IMP|Y<cr>

LRL|3A^RM17^17-2^FAC1|||LAB|3WEST PATH LAB<cr>

LDP|3A^RM17^17-2^FAC1|PED|MED|PIN|I|A|19941004||||(206)689-1363<cr>

LCC|3A^RM17^17-2^FAC1|PED|PIC|R38746<cr>

## CHARGE DESCRIPTION MASTER FILES

### Charge description master file message (MFN/MFK)

The charge description (CDM) master file segment should be used in conjunction with the general master file segments in Section 8.4, “general master file SEGMENTs.”  Interfacing systems often need not only to communicate data about a patient’s detailed charges, but also to communicate the charge identification entries by which an application knows how to handle a particular charge code.  The charge description master is a master file.  The CDM segment below is a specially designed master file segment for interfacing charge description masters.  In the following message, the MFI-master file identifier should equal “CDM.”  When the CDM segment is used in an MFN message, the abstract definition is as follows:

MFN					Master File Notification	Chapter

MSH				Message Header	2

MFI				Master File Identification	8

{MFE			Master File Entry	8

CDM			Charge Description Master	8

{ [PRC	] }		Price Segment	8

}

MFK					Master File Acknowledgment	Chapter

MSH				Message Header	2

MSA				Acknowledgment	2

MFI				Master File Identification	8

{ [MFA] }		Master File ACK segment	8

Master File Response Message:  When the CDM segment is used in the MFR message, the part of the message represented by:

{MFE

[Z..] }

is replaced by:

{MFE

CDM

{ [PRC] }

}

### CDM -  charge description master segment

The CDM segment contains the fields for identifying anything which is charged to patient accounts, including procedures, services, supplies.  It is intended to be used to maintain a list of valid chargeable utilization items.  Its purpose is to keep billing codes synchronized between HIS, Patient Accounting, and other departmental systems.  It is not intended to completely support materials management, inventory, or complex pricing structures for which additional complex fields would be required.  Given an identifying charge code, the associated fields in the charge description master file will provide basic pricing and billing data.  All the additional information necessary for patient accounting systems to do billing and claims is not intended to be included in this segment; those should be part of insurance or billing profile tables.

The CDM segment contains the fields which, for one chargeable item, remain the same across facilities, departments, and patient types.  The following PRC segment contains the fields which, for the same chargeable item, vary depending upon facility or department or patient type.

Figure 8-18.  CDM attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 200 | CE | R |  | 0132 | 01306 | Primary Key Value - CDM |
| 2 | 200 | CE | O | Y |  | 00983 | Charge Code Alias |
| 3 | 20 | ST | R |  |  | 00984 | Charge Description Short |
| 4 | 250 | ST | O |  |  | 00985 | Charge Description Long |
| 5 | 1 | IS | O |  | 0268 | 00986 | Description Override Indicator |
| 6 | 60 | CE | O | Y |  | 00987 | Exploding Charges |
| 7 | 80 | CE | O | Y | 0088 | 00393 | Procedure Code |
| 8 | 1 | ID | O |  | 0183 | 00675 | Active/Inactive Indicator |
| 9 | 60 | CE | O | Y |  | 00990 | Inventory Number |
| 10 | 12 | NM | O |  |  | 00991 | Resource Load |
| 11 | 200 | CK | O | Y |  | 00992 | Contract Number |
| 12 | 200 | XON | O |  | Y | 000993 | Contract Organization |
| 13 | 1 | ID | O |  | 0136 | 00994 | Room Fee Indicator |

8.9.2.0	CDM field definitions

#### Primary key value - CDM  (CE)   01306

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the code assigned by the institution for the purpose of uniquely identifying the thing that can be charged.  The key field of the entry.  For example, this field would be used to uniquely identify a procedure, item, or test for charging purposes.  Probably the same set of values as used in FT1-7 transaction code in financial messages.  Must match MFE-4-primary key value-MFE.  Refer to user-defined table 0132 - Transaction code.  See Chapter 7 for discussion of the universal service ID.

#### Charge code alias  (CE)   00983

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains an alternative charge code.  For example, points to another charge description master entry in cases where one code supersedes or overrides another code.  Repeating field allows for different codes used by different systems which should be handled as if they were the same; for example, the general ledger code may differ from the billing code.  Or, in a multi-facility environment which does facility-specific pricing, there may be more than one of these master file entries for one charge description, each with a different facility.

#### Charge description short  (ST)   00984

Definition:  This field contains the text abbreviations or code that is associated with this CDM entry.

#### Charge description long  (ST)   00985

Definition:  This field contains the full text description of this CDM entry.

#### Description override indicator  (IS)   00986

Definition:  This field indicates whether this CDM entry’s description can be overridden.  Refer to user-defined table 0268 - Override for suggested values.

User-defined Table  0268 - Override

| Value | Description |
| --- | --- |
| X | Override Not Allowed |
| A | Override Allowed |
| R | Override Required |

#### Exploding charges  (CE)   00987

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the repeating occurrences for a list of other CDM entry charge codes identifying the other charges which should be generated from this CDM entry.  If non-null, posting a charge to this CDM entry should result in posting the charges identified here.  These are sometimes called “linked items.”

In the case of “chained” charges where the “lead” charge must be included in the exploded charges, the “lead” charge should be included in the list of exploding charges.  If the price of this parent charge is included in the message, then it overrides the sum of the exploded charges prices.

#### Procedure code  (CE)   00393

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the procedure code for procedure, if any, associated with this charge description.  Repeating field allows for different procedure coding systems such as CPT4, ASTM, ICD9.  Coded entry made up of code plus coding schema.  Refer to user defined table 0088 - Procedure code for suggested values.

#### Active/inactive indicator  (ID)   00675

Definition:  This field indicates whether this is a usable CDM entry.  Refer to HL7 table 0183 - Active/inactive for valid values.

#### Inventory number  (CE)  00990

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This optional field contains an identifying stock number, if any, which might be used, for example, as a cross reference for materials management.

#### Resource load  (NM)  00991

Definition:  This field contains the Relative Value Unit (RVU) minutes and ATS, a factor related to CPT4 coding and to pricing structure for physical billing.

#### Contract number  (CK)   00992

Components:  <ID number (NM)> ^ <check digit (NM)> ^ <code identifying the check digit scheme employed  (ID)> ^ <assigning authority (HD)>

Subcomponents of assigning authority:  <namespace ID (IS)> & <universal ID (ST) & <universal ID Type (ID)

Definition:  This field contains any contract number pertaining to this chargeable item.  For example, supplier contract or service contract.

#### Contract organization  (XON)   00993

Components:	<organization name (ST)> ^ <organization name type code (ID)> ^ <ID number (ID)> ^ <check digit (NM)> ^ < check digit scheme (ID)> ^ <assigning authority (HD)> ^ <identifier type code (ID)> ^ <assigning facility ID (HD)> ^ <name representation code)

Subcomponents of assigning authority:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Subcomponents of assigning facility ID:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Definition:  This field contains the organization with whom there is a contractual arrangement for providing the service or material used for this chargeable item.

#### Room fee indicator  (ID)   00994

Definition:  This field contains a room fee indicator.  Refer to HL7 Table 0136-Yes/no indicator for valid values.

Y	this is a component of the room fees

N	this is any other chargeable item other than room fees

### PRC -  pricing segment

The PRC segment contains the  pricing information for the preceding CDM segment’s chargeable item.  It contains the fields which, for the same chargeable item, might vary depending upon facility or department or patient type.  The preceding CDM segment contains the fields which, for one chargeable item, remain the same across facilities, departments, and patient types.

Figure 8-19.  PRC attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 200 | CE | R |  | 0132 | 00982 | Primary Key Value - PRC |
| 2 | 60 | CE | O | Y |  | 00995 | Facility ID - PRC |
| 3 | 200 | CE | O | Y | 0184 | 00996 | Department |
| 4 | 1 | IS | O | Y | 0004 | 00967 | Valid Patient Classes |
| 5 | 12 | CP | C | Y |  | 00998 | Price |
| 6 | 200 | ST | O | Y |  | 00999 | Formula |
| 7 | 4 | NM | O |  |  | 01000 | Minimum Quantity |
| 8 | 4 | NM | O |  |  | 01001 | Maximum Quantity |
| 9 | 12 | MO | O |  |  | 01002 | Minimum Price |
| 10 | 12 | MO | O |  |  | 01003 | Maximum Price |
| 11 | 26 | TS | O |  |  | 01004 | Effective Start Date |
| 12 | 26 | TS | O |  |  | 01005 | Effective End Date |
| 13 | 1 | IS | O |  | 0268 | 01006 | Price Override Flag |
| 14 | 60 | CE | O | Y | 0293 | 01007 | Billing Category |
| 15 | 1 | ID | O |  | 0136 | 01008 | Chargeable Flag |
| 16 | 1 | ID | O |  | 0183 | 00675 | Active/Inactive Flag |
| 17 | 12 | MO | O |  |  | 00989 | Cost |
| 18 | 1 | IS | O |  | 0269 | 01009 | Charge On Indicator |

8.9.3.0	 PRC fields definitions

#### Primary key value - PRC  (CE)   00982

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the code assigned by the institution for the purpose of uniquely identifying the thing that can be charged.  The key field of the entry.  For example, this field would be used to uniquely identify a procedure, item, or test for charging purposes.  Probably the same set of values as used in FT1-7 transaction code in financial messages.  Must match MFE-4-primary key-MFE and CDM-1-primary key-CDM.  Refer to user-defined table 0132 - Transaction code.  See Chapter 7 for discussion of the universal service ID.

#### Facility ID - PRC  (CE)   00995

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the facility of the institution for which this price (for the preceding CDM entry) is valid.  For use when needing multi-facility pricing.  If null, assume all facilities.  In a multi-facility environment, the facility associated with this chargeable item may not be the same as the sending or receiving facility identified in the MSH segment.  Use only when the price is not the same for all facilities, that is, a null value indicates that this pricing is valid for all facilities.

When two PRC segments are sent with the same key values but different facility identifiers, the second is sent in addition to the first, not to replace the first.  The effective unique identifier is the charge code (PRC-1-primary key value-PRC) plus the facility ID (PRC-2-facility ID).  Multiple facility identifiers can be sent in the same segment to indicate that those facilities use the same pricing.

#### Department  (CE)   00996

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the department of the facility which accrues revenue/cost for this type of charge.  When pricing is different for different departments within the same facility, this will indicate for which department the following pricing information is valid.  Use only when the price is not the same for all departments, that is, a null value indicates that this pricing is valid for all departments.

When two PRC segments are sent the same key values but with different departments, the second is sent in addition to the first, not to replace the first.  The effective unique identifier is the charge code (PRC-1-primary key-PRC) plus the facility ID (PRC-2-facility ID) plus the department (PRC-3-department).  Multiple departments can be sent in the same segment to indicate that those departments use the same pricing. Refer to user-defined table 0184 - Department for suggested values.

#### Valid patient classes  (IS)   00967

Definition:  This field contains the patient types for which this charge description is valid.  For example, Inpatient, Outpatient, Series, Clinic, ER, Ambulatory, Observation, etc.  These values should be the same set of values as those used for PV1-3-patient class, which is site defined.  Use only when the price is not valid for all patient types, that is, a null value indicates that this pricing is valid for all patient classes.  Refer to user-defined table 0004 - Patient class for suggested values.

When two PRC segments are sent the same key values but with different valid patient classes, the second is sent in addition to the first, not to replace the first.  The effective unique identifier is the charge code (PRC-1-PRC primary key) plus the facility ID (PRC-2-facility ID) plus the department (PRC-3-department) plus the patient class (PRC-4-valid patient classes).  Multiple patient classes can be sent in the same segment to indicate that those patient classes use the same pricing.

#### Price  (CP)   00998

Components:  <price (MO)> ^ <price type (ID)> ^ <from value (NM)> ^ <to value (NM)> ^ <range units (CE)> ^ <range type (ID)>

Subcomponents of price:  <quantity (NM)> & <denomination (ID)>

Subcomponents of range nits:  <identifier (ST)> & <text (ST) & <name of coding system (ST)> & <alternate identifier (ST)> & <alternate text (ST)> & <name of alternate coding system (ST)>

Definition:  This field contains the price to be charged for service, item, or procedure.  If CDM price will always be overridden when charges are posted, then this field is optional.  Otherwise, price would be a required field.  The formula or calculation that is to be used to get total price from these price components is left to implementation negotiations agreed upon by the participating institutions.  See Chapter 2, Section 2.8.8, “CP - composite price,” for a description of the use of the composite price (CP) data type.

#### Formula  (ST)   00999

Definition:  This field contains the mathematical formula to apply to PRC-5-price in order to compute total price.  The syntax of this formula must conform to Arden Syntax rules.

#### Minimum quantity  (NM)   01000

Definition:  This field contains the minimum number of identical charges allowed on one patient account for this CDM entry.

#### Maximum quantity  (NM)   01001

Definition:  This field contains the maximum number of identical charges allowed on one patient account for this CDM entry.

#### Minimum price  (MO)   01002

Components:  <quantity (NM)> ^ <denomination (ID)>

Definition:  This field contains the minimum total price (after computation of components of price) that can be charged for this item.

#### Maximum price  (MO)   01003

Components: <quantity (NM)> ^ <denomination (ID)>

Definition:  This field contains the maximum total price (after computation of components of price) that can be charged for this item.

#### Effective start date  (TS)   01004

Definition:  This field contains the date/time when this CDM entry becomes effective.

#### Effective end date  (TS)   01005

Definition:  This field contains the date/time when this CDM entry is no longer effective.

#### Price override flag  (IS)   01006

Definition:  This field indicates whether this CDM entry’s price can be overridden.  Refer to user-defined table 0268 - Override for suggested values.

#### Billing category  (CE)   01007

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains the billing category codes for any classification systems needed, for example, general ledger codes and UB92 categories.  Repeating field with coded entry made up of category code plus category system.  Refer to user-defined table 0293 - Billing category for suggested values.

#### Chargeable flag  (ID)   01008

Definition:  This field contains a chargeable indicator. Refer to HL7 table 0136 - Yes/no Indicator for valid values.

N	charge is not billable, that is, do not create charges for this CDM entry; this is zero price item

Y	item is billable (this is also the default when NULL)

#### Active/inactive flag  (ID)   00675

Definition:  This indicates whether this is a usable CDM entry.  Refer to HL7 table 0183 - Active/inactive for valid values.

#### Cost  (MO)   00989

Components:  <quantity (NM)> ^ <denomination (ID)>

Definition:  This field contains the institution’s calculation of how much it costs to provide this item, that is, what the institution had to pay for the material plus any specified payment expenditure, effort or loss due to performing or providing the chargeable item.

#### Charge on indicator  (IS)   01009

Definition:  This field contains the user-defined table of values which indicates when a charge for services or procedures should be accrued.  Refer to user-defined table 0269 - Charge on indicator for suggested values.

User-defined Table  0269 - Charge on indicator

| Value | Description |
| --- | --- |
| O | Charge on Order |
| R | Charge on Result |

### Example:  MRN message charge description master file

MSH|^~\&|HL7REG|UH|HL7LAB|CH|19910918060544||MFN^M04|MSGID002|P|2.2||AL|NE<cr>

MFI|CDM||UPD|||AL<cr>

MFE|MAD|CDM98123789182|199110011230|P2246^^PLW<cr>

CDM|P2246^^PLW |2445||APPENDECTOMY|X||244.34|A|2321||||N<cr>

PRC|P2246^^PLW |FAC3|SURG|O~A|100.00^UP |formula |1|1 	|100.00^USD|1000.00^USD|19941031||Y|GL545|Y|A|<cr>

## Clinical Trials MASTER FILES

### Clinical trials master file message (MFN/MFK)

The CM0 (Clinical Study Master), CM1 (Clinical Study Phase), and CM2 (Clinical Study Schedule) segments can be used to transmit master files information between systems. The CM0 segment contains the information about the study itself; the CM1 contains the information about one phase of the study identified in the preceding CM0; and the CM2 contains the information about the scheduled time points for the preceding study or phase-related treatment or evaluation events.  When these segments are used in an MFN message, the abstract definition is described below.

Case 1: MFN message for Clinical Study with phases and schedules

MFI-1-master file identifier code = CMA

MFN					Master File Notification	Chapter

MSH					Message Header	2

MFI					Master File Identification	8

{ MFE				Master File Entry	8

CM0				Clinical Study Master	8

[{ CM1			Clinical Study Phase	8

[{CM2}] }]		Clinical Study Schedule	8

}

MFK					Master File Acknowledgment	Chapter

MSH					Message Header	2

MSA					Acknowledgment	2

MFI					Master File Identification	8

{ [MFA] }			Master file ACK 	8

Case 2: MFN message for Clinical Study without phases but with schedules

MFI-1-master file identifier code = CMB

MFN					Master File Notification	Chapter

MSH					Message Header	2

MFI					Master File Identification	8

{ MFE				Master File Entry	8

CM0				Clinical Study Master	8

[ {CM2}]		Clinical Study Schedule	8

}

MFK					Master File Acknowledgment	Chapter

MSH					Message Header	2

MSA					Acknowledgment	2

MFI					Master File Identification	8

{ [MFA] }			Master file ACK	8

When the Clinical Trials master segments are used in the MFR message, the part of the message represented by:

{MFE

[Z..] }

is replaced by, in case 1 above:

{ MFE

CM0

[{ CM1

[ {CM2}]

}

In case 2 above, the corresponding segments in the MFR message represented by:

{MFE

[Z..] }

are replaced by

{ MFE

CM0

[ {CM2}] }]

}

### CM0 - clinical study master segment

The Clinical Study Master (CM0) segment contains the information about the study itself.  The sending application study number for each patient is sent in the CSR segment.  The optional CM0 enables information about the study at the sending application that may be useful to the receiving systems.  All of the fields in the segment describe the study status at the sending facility unless otherwise agreed upon.

Figure 8-20.  CM0 attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | SI | O |  |  | 01010 | Set ID - CM0 |
| 2 | 60 | EI | R |  |  | 01011 | Sponsor Study ID |
| 3 | 60 | EI | O | Y/3 |  | 01012 | Alternate Study ID |
| 4 | 300 | ST | R |  |  | 01013 | Title of Study |
| 5 | 60 | XCN | O | Y |  | 01014 | Chairman of Study |
| 6 | 8 | DT | O |  |  | 01015 | Last IRB Approval Date |
| 7 | 8 | NM | O |  |  | 01016 | Total Accrual to Date |
| 8 | 8 | DT | O |  |  | 01017 | Last Accrual Date |
| 9 | 60 | XCN | O | Y |  | 01018 | Contact for Study |
| 10 | 40 | XTN | O |  |  | 01019 | Contact's Tel. Number |
| 11 | 100 | XAD | O | Y |  | 01020 | Contact's Address |

8.10.2.0  CM0 field definitions

#### Set ID - CM0 (SI)   01010

Definition:  This field contains a number that uniquely identifies this transaction for the purpose of adding, changing, or deleting the transaction.  For those messages that permit segments to repeat, the  Set ID field is used to identify the repetitions.

#### Sponsor study ID  (EI)   01011

Components:  <entity identifier (ST)> ^ <namespace ID (IS)> ^ <universal ID (ST)> ^ <universal ID type (ID)>

Definition:  This field contains the study number established by the study sponsor.  Please see discussion in Section 7.7.1.1, “Sponsor study ID.”

#### Alternate study ID  (EI)   01012

Components:  <entity identifier (ST)> ^ <namespace ID (IS)> ^ <universal ID (ST)> ^ <universal ID type (ID)>

Definition:  This field contains the local or collaborators’ cross-referenced study numbers.

#### Title of study  (ST)   01013

Definition:  This field contains the sending institution’s title for the clinical trial.  It gives recipients further identification of the study.

#### Chairman of study (XCN)   01014

Components:	<ID number (ST)> ^ <family name (ST)> & <last name prefix (ST)> ^ <given name (ST)> ^ <middle initial or name (ST)> ^ <suffix (e.g., JR or III) (ST)> ^ <prefix (e.g., DR) (ST)> ^ <degree (e.g., MD) (ST)> ^ <source table (IS)> ^ <assigning authority (HD)> ^ <name type (ID)> ^ <identifier check digit (ST)> ^ <code identifying the check digit scheme employed (ID) ^ <identifier type code (IS)> ^ <assigning facility ID (HD)> ^ <name representation code>

Subcomponents of assigning authority:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Subcomponents of assigning facility ID:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Definition:  This field contains the sending institution’s chairman.  It further identifies the study.  The chairman’s name may be needed for communication purposes.

#### Last IRB approval date  (DT)   01015

Definition:  This field contains an institution’s Internal Review Board approval dates which are required annually to continue participation in a clinical trial.

#### Total accrual to date  (NM)   01016

Definition:  This field is a quality control field to enable checks that patient data have been transmitted on all registered patients.

#### Last accrual date  (DT)   01017

Definition:  This field contains the status information on the patient registration activity for quality control and operations purposes.

#### Contact for study  (XCN)   01018

Components:	<ID number (ST)> ^ <family name (ST)> & <last name prefix (ST)> ^ <given name (ST)> ^ <middle initial or name (ST)> ^ <suffix (e.g., JR or III) (ST)> ^ <prefix (e.g., DR) (ST)> ^ <degree (e.g., MD) (ST)> ^ <source table (IS)> ^ <assigning authority (HD)> ^ <name type (ID)> ^ <identifier check digit (ST)> ^ <code identifying the check digit scheme employed (ID) ^ <identifier type code (IS)> ^ <assigning facility ID (HD)> ^ <name representation code>

Subcomponents of assigning authority:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Subcomponents of assigning facility ID:  <namespace ID (IS)> & <universal ID (ST)> & <universal ID type (ID)>

Definition:  This field contains the name of the individual who should be contacted for inquiries about data transmitted for this study.

#### Contact’s telephone number (XTN)   01019

Components: [NNN] [(999)]999-9999 [X9999] [C any text] ^ <telecommunication use code (ID)> ^ <telecommunication equipment type (ID)> ^ <email address (ST)> ^ <county code (NM)> ^ <area/city code (NM)> ^ <phone number (NM)> ^ <extension (NM)> ^ <any text (ST)>

Definition:  This field contains the phone number of the study contact identified in CM0-9-contact for study.

#### Contact's address  (XAD)   01020

Components:	<street address (ST)> ^ <other designation (ST)> ^ <city (ST)> ^ <state or province (ST)> ^ <zip or postal code (ST)> ^ <country (ID)> ^ <address type (ID)> ^ <other geographic designation (ST)> ^ <county/parish code (IS)> ^ <census tract (IS)> ^ <address representation code (ID)>

Definition:  This field contains the address of the study contact identified in CM0-9-contact for study.

### CM1 - clinical study phase master segment

Each Clinical Study Phase Master (CM1) segment contains the  information about one phase of a study identified in the preceding CM0.  This is an optional structure to be used if the study has more than one treatment or evaluation phase within it.  The identification of study phases that the patient enters are sent in the CSP segment: sequence 2.  The CM1 segment describes the phase in general for the receiving system.

Figure 8-21.  CM1 attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | SI | R |  |  | 01021 | Set ID - CM1 |
| 2 | 60 | CE | R |  |  | 01022 | Study Phase Identifier |
| 3 | 300 | ST | R |  |  | 01023 | Description of Study Phase |

8.10.3.0   CM1 field definitions

#### Set ID - CM1  (SI)   01021

Definition:  This field contains a number that uniquely identifies this transaction for the purpose of adding, changing, or deleting the transaction.  For those messages that permit segments to repeat, the  Set IF field is used to identify the repetitions.

#### Study phase identifier  (CE)   01022

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field should correspond to the study phase ID coding system in Section 7.7.2.1, “Study phase ID.”

#### Description of study phase  (ST)   01023

Definition:  This field contains a brief explanation for recipients to understand what the phase represents.

### CM2 - clinical study schedule master segment

The Clinical Study Schedule Master (CM2) contains the  information about the scheduled time points for study or phase-related treatment or evaluation events.  The fact that a patient has data satisfying a scheduled time point is sent in the CSS segment, sequence 2.  The CM2 segment describes the scheduled time points in general.

Figure 8-22.  CM2 attributes

| SEQ | LEN | DT | OPT | RP/# | TBL# | ITEM# | ELEMENT NAME |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | SI | O |  |  | 01024 | Set ID- CM2 |
| 2 | 60 | CE | R |  |  | 01025 | Scheduled Time Point |
| 3 | 300 | ST | O |  |  | 01026 | Description of Time Point |
| 4 | 60 | CE | R | Y/200 |  | 01027 | Events Scheduled This Time Point |

8.10.3.0   CM2 field definitions

#### Set ID - CM2  (SI)   01024

Definition:  This field contains a number that uniquely identifies this transaction for the purpose of adding, changing, or deleting the transaction.  For those messages that permit segments to repeat, the  Set ID field is used to identify the repetitions.

#### Scheduled time point  (CE)   01025

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field should correspond to the scheduled time point coding system in Section 7.7.3.1, “Study scheduled time point.”

#### Description of time point  (ST)   01026

Definition:  This field contains a brief explanation so recipients will understand what the time point represents.

#### Events scheduled this time point  (CE)   01027

Components:  <identifier (ST)> ^ <text (ST)> ^ <name of coding system (ST)> ^ <alternate identifier (ST)> ^ <alternate text (ST)> ^ <name of alternate coding system (ST)>

Definition:  This field contains a study-specific event.  Coding systems may be developed for this field or applications may use facility-wide or standardized orders and procedures coding systems.  This enables integration of procedures or events ordered for clinical trials with medical order entry systems.

## OUTSTANDING ISSUES

### We invite proposals for the specification of other HL7-wide master files segments.

International Union of Pure and Applied Chemistry/International Federation of Clinical Chemistry.  The Silver Book: Compendium of terminology and nomenclature of properties in clinical laboratory sciences. Oxford: Blackwell Scientific Publishers, 1995.

NCCLS Document H1-A3: Evacuated tubes for blood specimen collection -- Third Edition, Volume 11, Number 9, Approved standard. July 1991.